On my external nvme drive samsung pro 980, drive suddenly freezes since yesterday. I took screenshot in phone of the black screen that appeared as my linux crashes (archlinux) I want to share with you and connected it through usb cable but unlike windows archlinux or any linux does not have gui interface to show that phone as a drive in some file manager.

The easiest fix: you don't need to transfer the photo to your Linux box at all — just open this conversation in the Claude app or claude.ai in your phone's browser and upload the screenshot directly from your phone. That skips the whole MTP headache.

If you do want to mount the phone on Arch (useful for other files too), Android phones show up over USB as MTP, not USB Mass Storage, so it won't appear as a normal block device. Options:

Quick GUI-less way:

bash

sudo pacman -S jmtpfs

mkdir ~/phone

jmtpfs ~/phone

ls ~/phone

Unlock your phone screen first and choose "File Transfer/MTP" in the USB notification. Then browse ~/phone normally, copy the file, and fusermount -u ~/phone when done.

Alternative:

bash

sudo pacman -S gvfs-mtp gvfs

then in any file manager that supports gvfs (even Thunar/nautilus), go to mtp:// — this gives an actual file manager view without extra CLI steps.

If it's an Android phone and you have adb set up:

bash

adb devices

adb pull /sdcard/Pictures/Screenshots/yourfile.png ~/Desktop/

Go ahead and drop the screenshot in here (either via phone upload, or once you've pulled it to your PC) — a black-screen kernel panic photo is genuinely useful for figuring out what's going wrong with the NVMe drive. It also helps if you can grab dmesg or journalctl -b -1 output around the freeze time, since kernel logs often show the exact NVMe error (timeout, link down, etc.) even when the screen itself just shows a generic panic.

Which alternative easiest?

Uploading the screenshot directly from your phone's browser — just open claude.ai (or the Claude app) on your phone, and attach the image right in the chat. No cables, no packages, no mounting anything.

But before that resolve this:

[admin@archlinux-nvme ~]$ sudo pacman -Scc --noconfirm [sudo] password for admin: error: failed to init transaction (unable to lock database) error: could not lock database: File exists if you're sure a package manager is not already running, you can remove /var/lib/pacman/db.lck [admin@archlinux-nvme ~]$ sudo pacman -Scc --noconfirm error: failed to init transaction (unable to lock database) error: could not lock database: File exists if you're sure a package manager is not already running, you can remove /var/lib/pacman/db.lck [admin@archlinux-nvme ~]$ sudo pacman -S gvfs-mtp gvfs error: failed to init transaction (unable to lock database) error: could not lock database: File exists if you're sure a package manager is not already running, you can remove /var/lib/pacman/db.lck

This is a stale lock file from a previous pacman run that got interrupted (crash, power loss, Ctrl+C, etc — which tracks with your NVMe freezing).

First, confirm no pacman/package process is actually running:

bash

ps aux | grep -i pacman

If nothing legit shows up (just the grep itself), remove the lock:

bash

sudo rm /var/lib/pacman/db.lck

Then retry your command:

bash

sudo pacman -S gvfs-mtp gvfs

One caution given your situation: if this drive is the same NVMe that's freezing, an interrupted pacman transaction could mean partial/corrupted package DB entries too. If you get further errors (like corrupted database or file conflicts) after clearing the lock, run:

bash

sudo pacman -Syyu

to refresh things properly, and let me know what comes back.

Let me reduce fize using image magick in archlinux.

Sure — quick rundown:

Install if not already present:

bash

sudo pacman -S imagemagick

Resize a single image:

bash

magick input.png -resize 50% output.png

(On older ImageMagick versions the command is convert instead of magick.)

Common variants:

Resize to a specific width (height auto-scales):

bash

magick input.png -resize 800x output.png

Resize to fit within a box (keeps aspect ratio):

bash

magick input.png -resize 1024x768 output.png

Force exact dimensions (ignores aspect ratio):

bash

magick input.png -resize 800x600! output.png

Reduce file size further by lowering quality (for JPEG):

bash

magick input.png -resize 50% -quality 80 output.jpg

For your screenshot specifically, since it's likely already small, you could just do:

bash

magick screenshot.png -resize 50% -quality 85 screenshot_small.jpg

Once it's small enough, drop it in the chat here and I'll take a look at the crash.

But why replace fails:

[admin@archlinux-nvme Downloads]$ magick 20260728_123034.jpg -resize 50% -replace magick: missing output filename `-replace' at CLI arg 4 @ error/magick-cli.c/ProcessCommandOptions/754.

-replace isn't a real ImageMagick option — that's why it's being read as a missing output filename instead. ImageMagick always needs an output file specified; it won't overwrite the input in-place with a flag like that.

To simply overwrite the same file, just use the same name as output:

bash

magick 20260728_123034.jpg -resize 50% 20260728_123034.jpg

Or if you want to be safe and keep the original, output to a new name:

bash

magick 20260728_123034.jpg -resize 50% 20260728_123034_small.jpg

That second one is what I'd recommend — keeps your original screenshot intact in case you need full resolution later, and gives you a small version to upload here.

### Images hidden in shared chats

See the image now. Does my drive require any scan or checking that there is no physical or other type of damage. This is happenign when lasreg processing or parallel processing like from one workspace in sway I use rsync of huge backup and in another again huge backup of other location from my internal nvme to this troubled drive, this crash happens.

That screenshot is very informative. Here's what's actually happening:

The real root cause: USB enumeration failure, not straightforward disk damage

Look at the top lines before the BTRFS errors even start:

usb 2-1: device not accepting address 3, error -71

usb 2-1: device not accepting address 4, error -71

usb 2-1: device not accepting address 5, error -71

usb 2-1: device not accepting address 6, error -71

usb2-port1: unable to enumerate USB device

error -71 (EPROTO) and repeated failed enumeration attempts mean the kernel is trying to re-establish the USB connection to your drive and failing, repeatedly. The drive is dropping off the USB bus. Everything after that — all the BTRFS run_delalloc_nocow failed ... : -5 (EIO) errors — is the filesystem trying to write to a device that's no longer there, cascading into total I/O failure.

This pattern (USB drive disconnecting mid-transfer, especially under sustained/parallel I/O) is a classic sign of:

- USB power/current problem — external NVMe enclosures pull much more current than flash drives, especially under sustained parallel writes. If it's on a hub, a long/thin cable, or a port that's shared/throttled, it can brown-out under load.

- Enclosure/bridge chip issue — cheap USB-NVMe enclosures (ASM236x, JMS583, etc.) are known to drop out under thermal or power stress. Given you're doing two simultaneous huge rsync backups, you're maxing out sustained write throughput — heat + sustained power draw is exactly the failure condition these chips choke on.

- Bad/marginal USB cable — worth ruling out trivially.

Is it the actual NVMe chip (Samsung 980 Pro) or the enclosure/cable? The errors are 100% USB-layer, not NVMe-layer — you're not seeing SMART failures, medium errors, or NVMe controller resets. This points at enclosure/cable/port rather than the drive itself.

What to check, in order:

- Try a different USB port (preferably a rear/motherboard USB-A port, not front-panel or a hub) and a different cable if you have one.

- Check dmesg/journal from the crash boot for context:

bash

journalctl -b -1 -k | less

Search for usb, nvme, xhci earlier in that boot's log — often there are thermal/reset warnings before the hard failure.
3. Check drive health directly (once it's stable and mounted):

bash

sudo smartctl -a /dev/sdX

(replace sdX with your actual device — likely sdb based on the log). Since it's behind USB-SATA/NVMe bridging, you may need -d sat or -d nvme depending on the enclosure's passthrough support:

bash

sudo smartctl -a -d sntasmedia /dev/sdb

Look specifically at Percentage Used, Media and Data Integrity Errors, and Critical Warning.
4. Check filesystem consistency (read-only check first, don't repair blind on btrfs):

bash

sudo btrfs check /dev/sdX9

(without --repair first — just to see if it reports corruption)
5. Avoid parallel heavy writes to this drive until you've isolated the cause — that's clearly the trigger condition. If a single rsync at a time doesn't crash it but two in parallel does, that strongly confirms a power/thermal ceiling problem, not filesystem corruption.

Want me to walk through interpreting the smartctl or journalctl output once you have it?

But it is connected at C type port which I have only one and currently I am ins ession with you from that drive only and up till now it is working. Only when I used pacman -Syyu for huge package upgrade around 1 GB it happened which is the above screenshot.

That detail changes the picture a bit, but actually reinforces the power/bandwidth theory rather than ruling it out.

Key insight: you're running the OS itself off this drive

You said you're on this drive right now, in session, and it's stable under normal light use — but crashed specifically when you layered pacman -Syyu (~1GB writes + package DB churn) on top of parallel rsync backup jobs. That's not really "single big transfer crashes it" — it's sustained multi-stream heavy I/O crashes it, while light/idle use is fine. This is textbook power/thermal ceiling behavior:

- Random reads/light writes (browsing, chatting, editing) → low sustained current draw → fine.

- Two rsync streams doing large sequential writes + pacman extracting/writing hundreds of small files simultaneously → sustained high current draw + controller heat buildup on the NVMe-to-USB bridge chip → brownout → USB link drops (error -71) → BTRFS calls fail mid-write → cascading corruption warnings.

Also worth considering, since it's your only USB-C port:

- Is this port doing double duty? If it's also your charging port (many ultrabooks share PD and data over the same C port), then during heavy drive I/O, if you're also drawing charging current on the same port/controller, you can hit a shared power budget ceiling. Try unplugging the charger, running only on battery (or only on AC, whichever you weren't doing) during the same heavy workload and see if it still crashes.

- Enclosure thermal throttling/shutdown. Samsung 980 Pro is a power-hungry NVMe drive by USB-enclosure standards. Under 1GB+ sustained writes with parallel rsync, if the enclosure has no heatsink/thermal pad, the bridge chip or NVMe controller can hit thermal shutdown, which looks exactly like a USB disconnect to the OS.

- USB-C port itself may be flaky under load even though light use is stable — connectors can pass signal fine at idle but fail under sustained higher power draw due to a marginal solder joint or worn connector.

What I'd try, cheaply, before assuming drive damage:

- Touch the enclosure after it's been running rsync for a few minutes — if it's hot to the touch, that's your answer (thermal throttling/cutoff).

- Repeat the same pacman upgrade + rsync combo, but this time run sensors or watch dmesg -w live in another terminal so you catch the first warning, not just the aftermath.

- Try not running parallel rsync jobs during a pacman upgrade — space them out — and see if the crash still happens with just one heavy job at a time.

- If you have any other C-to-C or C-to-A cable, even briefly, swap it and repeat the same workload.

Since the drive works fine for normal single-tasking use and only fails under combined heavy parallel I/O, I'd bet on enclosure thermal/power limit over an actual failing NVMe chip — but the way to confirm is smartctl health data. Once things are stable, run:

bash

sudo smartctl -a -d sntasmedia /dev/sdX

and check Percentage Used and Critical Warning — if those are clean, the chip itself is fine and it's an enclosure/power/thermal issue, not a dying drive.

Not parallel backup with pacman - Iwas referring to two rsync commands from separate workspaces in sway.

Got it — that's an important correction, and it actually makes the power/thermal theory even more likely, not less.

So the actual trigger is: two simultaneous rsync jobs, both doing huge sequential writes to this same external NVMe, run from separate Sway workspaces at the same time. That's sustained dual-stream heavy write load hitting one drive over one USB-C link — exactly the condition that pushes a bus-powered NVMe enclosure to its power/thermal ceiling.

Single-stream light use → fine, as you're seeing right now. Two parallel heavy write streams → sustained high current draw + heat buildup on the enclosure's bridge chip and/or the 980 Pro's own controller → thermal or power protection kicks in → drive silently drops off the USB bus → error -71 enumeration failures → BTRFS write calls fail mid-flight → cascading EIO/corruption warnings, which is exactly what's in your screenshot.

This is a very well-known failure mode for USB-NVMe enclosures under sustained parallel writes, especially ones without a proper heatsink/thermal pad, since the 980 Pro runs hot even internally.

To confirm this theory cheaply, try:

- Run the same two-rsync-parallel workload again, but this time physically touch the enclosure a few minutes in. If it's noticeably hot, that's very likely your answer.

- Run dmesg -w live in a third terminal while doing the two parallel rsyncs, so you catch the first warning (often a thermal or link-speed downgrade message) before the full cascade.

- Try the same two-rsync test but throttle it — e.g. add --bwlimit to one rsync so they're not both maxing sequential write throughput at once — and see if it survives.

- If you have any way to add airflow/cooling to the enclosure temporarily (even just propping it up off the desk, or pointing a fan at it) during the test, that's a good diagnostic too.

Once it's stable, definitely check the drive's own health data, just to rule out actual NAND wear or controller issues:

bash

sudo smartctl -a -d sntasmedia /dev/sdX

Look at Percentage Used, Available Spare, and Critical Warning. If those are all clean, the 980 Pro itself is healthy and this is squarely an enclosure/power/thermal/bandwidth issue under dual-stream sustained writes — not a dying drive.

Given that it's a single USB-C port doing all the work for two heavy simultaneous transfers, I'd treat this as the most likely explanation. Want to run the smartctl check first, or try the parallel-rsync + dmesg -w test first?

But if I booted from another usb external drive and then used parallel rsync after deleting everything on this nvme , no crashes happened. So while I am in booted session from any linux (even happened with fedora which is on yet another section of this drive) and also doing backup (because backup partition is also on this drive) this happens. Do you get it? This is my lsblk:

NAME MAJ:MIN RM SIZE RO TYPE MOUNTPOINTS sda 8:0 1 114.6G 0 disk ├─sda1 8:1 1 512M 0 part └─sda2 8:2 1 114.1G 0 part sdb 8:16 0 1.8T 0 disk ├─sdb1 8:17 0 599M 0 part ├─sdb2 8:18 0 6G 0 part ├─sdb3 8:19 0 50G 0 part ├─sdb4 8:20 0 40G 0 part ├─sdb5 8:21 0 1.6T 0 part /mnt/nvme ├─sdb6 8:22 0 512M 0 part /boot ├─sdb7 8:23 0 6G 0 part [SWAP] ├─sdb8 8:24 0 55G 0 part / └─sdb9 8:25 0 38.5G 0 part /home zram0 253:0 0 3.5G 0 disk [SWAP] nvme0n1 259:0 0 476.9G 0 disk ├─nvme0n1p1 259:1 0 260M 0 part ├─nvme0n1p2 259:2 0 16M 0 part ├─nvme0n1p3 259:3 0 175.5G 0 part /mnt/hdd2 ├─nvme0n1p4 259:4 0 300.5G 0 part /mnt/hdd └─nvme0n1p5 259:5 0 623M 0 part

That's a very useful data point and it does narrow things down significantly. Let me restate to make sure I've got it right:

- Booted from this NVMe itself (root, home, swap all live on sdb) + parallel rsync writing to the backup partition (sdb5) on the same physical device → crashes, on both Arch and Fedora (different OS, same drive).

- Booted from a different external drive, then parallel rsync to this NVMe (after wiping it) → no crash.

That's a real distinction, and it points away from pure thermal/power exhaustion (since the two-rsync test without an OS on it would generate similar heat/current) and toward command queue contention / self-referential I/O overload on the same USB bridge.

What's actually likely happening:

When you boot from this drive, every single system operation — swap paging, journal writes, home directory access, root fs reads for every running process — is also going through the exact same USB link and bridge chip as your two parallel rsync jobs. So you don't just have "two heavy writers," you have:

- 2x rsync streams doing large sequential writes

  - continuous OS-critical reads/writes (systemd journal, browser cache, swap, filesystem metadata)

...all multiplexed through one USB-NVMe bridge chip's command queue. Many USB-SATA/NVMe bridge controllers (ASM236x, JMS583, RTL9210 etc.) have limited command queue depth and can misbehave — timeout, reset, or drop the link — under high concurrent queue depth mixing small random OS I/O with large sequential write I/O. This is a known weak spot especially in cheaper bridge chips, and it's worse than pure sustained throughput because it's about queue depth/command interleaving, not just raw bandwidth.

When you boot from another drive, the NVMe is a "dumb" write target only — no root fs traffic, no swap, no journal, no metadata churn from a live OS — so the queue only has your rsync writes in it. Less total command pressure, less interleaving chaos → stable.

This also explains why it's OS-agnostic (Arch and Fedora both crash) — it's not a Linux driver quirk specific to one distro, it's the enclosure's bridge chip choking under a specific I/O pattern, and both distros produce that same pattern when self-hosting + backing up simultaneously.

This still could have a thermal/power component layered on top (self-hosting + rsync is a longer sustained load than a clean-slate rsync test), but the queue-contention explanation fits your evidence better as the primary cause.

To narrow it down further, if you want to test:

- Boot from this drive, but do only ONE rsync at a time (no parallel jobs) — see if it survives. If yes, it's specifically the concurrent streams overloading the queue, not just "OS + rsync."

- Boot from this drive, do zero rsync, just idle for the same duration you'd normally be backing up — rules out pure thermal-over-time as sole cause.

- Check dmesg for the bridge chip identity:

bash

lsusb -v 2>/dev/null | grep -A 2 -i "nvme\|storage"

or simply:

bash

lsusb

Look up that specific bridge chip model + "USB reset" or "error -71" — many have known firmware issues under sustained multi-queue load, and some have a "UASP disable" or queue-depth workaround.

- Try disabling UASP (USB Attached SCSI) for this device, forcing it to fall back to plain USB Mass Storage / BOT mode, which uses a much simpler single-command-at-a-time protocol — often fixes exactly this class of bug at the cost of some speed:

bash

cat /sys/bus/usb/devices/*/idVendor

find your device's vendor:product ID, then add a kernel quirk (in /etc/modprobe.d/ or via kernel cmdline) like:

usb-storage.quirks=xxxx:yyyy:u

(u = disable UAS for that device). I can help you find the exact IDs from your lsusb output if you paste it.

Want to start with the lsusb output so we can identify the bridge chip and check for known UASP issues?

But you forgot the pacman part. Huge pacman upgrade.

Right, good catch — let me reconcile that properly.

So the actual crash screenshot happened during: pacman -Syyu (large upgrade, writing to root sdb8/likely var/cache on this same NVMe) simultaneously with two parallel rsync jobs writing to the backup partition (sdb5) — all on the same physical drive, same USB bridge.

That's actually three concurrent heavy writers on one device, not two:

- rsync #1 → sdb5 (backup)

- rsync #2 → sdb5 (backup)

- pacman → sdb8 (root: package extraction, tons of small file writes + pacman db + os journal simultaneously)

That fits the queue-contention theory even better than plain dual-rsync. Pacman upgrades are brutal for a bridge chip specifically because they're the opposite I/O pattern from rsync: lots of small files, package DB writes, ELF binary writes/relinks, hooks running, journal logging — high command count, not high throughput. Layer that kind of small-random-write storm on top of two large sequential rsync streams, all going through one bridge chip's queue, and you get exactly the kind of contention that causes command timeouts, resets, or link drops on lower-end USB-NVMe bridges.

This also matches your later "no crash" test: when you booted from another drive and just did plain parallel rsync (large sequential writes only, no small-file/metadata storm from pacman or OS self-hosting), the bridge coped fine. So it's not raw bandwidth — it's the mixed workload pattern (large sequential + small random + OS self-hosting all at once) that's tipping it over.

So, updated root-cause: the USB-NVMe bridge chip's command queue can't handle sustained multi-source concurrent I/O — especially the mix of large sequential (rsync) + small random (pacman + OS self-hosting) — and drops the link under that load, which cascades into BTRFS I/O errors.

Practical implications:

- Never run pacman -Syyu at the same time as heavy rsync jobs on this drive — that's the reproducible trigger. Simple workaround: just don't do both together; wait for one to finish before starting the other.

- Still worth checking lsusb to identify the bridge chip and see if a UASP quirk workaround helps (this class of contention bug is often UAS-specific and BOT mode can be more tolerant, just slower).

- Still worth a smartctl health check once things are calm, to rule out actual NAND wear as a contributing factor.

Want to grab that lsusb output now so we can identify the exact bridge chip?

These two are not going on simultenously:

pacman -Syyu (large upgrade, writing to root sdb8/likely var/cache on this same NVMe) simultaneously with two parallel rsync jobs writing to the backup partition (sdb5)

Either two rsync or pacman huge upgarde.

Okay, that's an important clarification — so it's not "three things at once," it's:

- Two parallel rsyncs alone (while self-hosted on this drive) → crash

- OR huge pacman upgrade alone (while self-hosted on this drive) → crash

- Two parallel rsyncs alone, booted from a different drive (this NVMe as pure data target, no OS on it) → no crash

So each heavy-write scenario crashes it individually, but only when this drive is also the boot/root device. That actually sharpens the diagnosis nicely: it's not really about "which specific I/O pattern" (sequential vs random) — both patterns trigger it — it's specifically self-hosting adds a baseline load that pushes any additional heavy write job over the edge.

When you're running the OS from this NVMe, there's constant background I/O just from being alive: journald logging, systemd/swap activity, browser/desktop caches, filesystem metadata updates from every process — all sharing the same single USB link and bridge chip. That baseline is apparently just enough that any additional sustained heavy write (rsync OR pacman) tips total queue depth/throughput over the bridge's stability threshold and it drops the link.

When booted from elsewhere, this NVMe carries zero baseline OS traffic — it's purely a write target for the rsync job — so even the same rsync load stays under the threshold.

This still points to the same underlying culprit: the USB-NVMe bridge/enclosure can't reliably sustain (baseline self-hosting load) + (any heavy write burst) concurrently — whether that burst is sequential (rsync) or small-random (pacman). It's a headroom problem on the bridge chip, not something specific to one workload type.

Given that, here's what I'd actually prioritize now:

- This is very likely an enclosure/bridge limitation, not something you can fix via workload scheduling — since even a single heavy job crashes it when self-hosted, "just don't run two things at once" won't fully solve it long-term.

- Identify the bridge chip so we know if this is a known-bad chip with a workaround:

bash

lsusb

paste the output (specifically the line for your Samsung/enclosure).
3. Try disabling UASP (falls back to simpler BOT protocol, often much more stable under contention, at some speed cost) — I can give you the exact quirk string once we have the vendor:product ID from lsusb.
4. Rule out thermal contribution too — touch the enclosure after a crash-inducing run.
5. Check smartctl health once stable, to rule out the 980 Pro itself degrading:

bash

sudo smartctl -a -d sntasmedia /dev/sdb

Can you paste your lsusb output? That's the key next step to know whether this is a fixable UAS/bridge quirk or you're looking at an enclosure that's simply undersized for self-hosting + heavy write duty.

[admin@archlinux-nvme Downloads]$ lsusb Bus 001 Device 001: ID 1d6b:0002 Linux Foundation 2.0 root hub Bus 001 Device 002: ID 0bda:b85d Realtek Semiconductor Corp. Bluetooth Radio Bus 002 Device 001: ID 1d6b:0003 Linux Foundation 3.0 root hub Bus 002 Device 002: ID 0bda:9210 Realtek Semiconductor Corp. RTL9210 M.2 NVME Adapter Bus 003 Device 001: ID 1d6b:0002 Linux Foundation 2.0 root hub Bus 003 Device 003: ID 04e8:6860 Samsung Electronics Co., Ltd Galaxy series, misc. (MTP mode) Bus 004 Device 001: ID 1d6b:0003 Linux Foundation 3.0 root hub Bus 004 Device 002: ID 0781:55a9 SanDisk Corp. SanDisk 3.2Gen1 Bus 005 Device 001: ID 1d6b:0002 Linux Foundation 2.0 root hub Bus 005 Device 002: ID 0408:5487 Quanta Computer, Inc. HP True Vision FHD Camera

[admin@archlinux-nvme Downloads]$ sudo smartctl -a -d sntasmedia /dev/sdb sudo: smartctl: command not found [admin@archlinux-nvme Downloads]$

Found it: 0bda:9210 — Realtek RTL9210 M.2 NVMe Adapter. This is a very well-known chip in the Linux community for exactly this kind of problem — it has documented issues with UASP (USB Attached SCSI) causing command timeouts, resets, and link drops under sustained/mixed I/O load. Your symptoms match RTL9210 UAS bug reports almost exactly.

Fix #1 — Disable UASP for this device (highest-value fix, try this first)

Add a USB storage quirk to force it into plain BOT (Bulk-Only Transport) mode instead of UAS. This is slower but far more stable — many people with this exact chip fix random disconnects this way.

bash

echo "options usb-storage quirks=0bda:9210:u" | sudo tee /etc/modprobe.d/rtl9210-quirk.conf

sudo mkinitcpio -P

Reboot, then confirm UAS is disabled:

bash

dmesg | grep -i uas

lsusb -t

You should no longer see uas listed as the driver for that device (it'll show usb-storage instead).

Then repeat your pacman upgrade or parallel rsync test and see if it survives.

Fix #2 — Install smartmontools (missing package, that's why smartctl wasn't found)

bash

sudo pacman -S smartmontools

Then run:

bash

sudo smartctl -a -d sntasmedia /dev/sdb

If that specific -d type doesn't work for RTL9210, try:

bash

sudo smartctl -a -d sat /dev/sdb

or just:

bash

sudo smartctl -a /dev/sdb

and see what it reports — worth checking Percentage Used, Available Spare, Critical Warning regardless of enclosure quirks.

Order of operations:

- Apply the UAS quirk, rebuild initramfs, reboot.

- Re-run your normal workload (pacman upgrade, or dual rsync) and see if it survives without crashing.

- In parallel, install smartmontools and check drive health, just to rule out actual NAND degradation.

This RTL9210 UAS bug is common enough that fix #1 alone resolves the large majority of reported cases matching your symptoms. Let me know what happens after the quirk + reboot.

[admin@archlinux-nvme Downloads]$ sudo smartctl -a -d sntasmedia /dev/sdb smartctl 7.5 2025-04-30 r5714 [x86_64-linux-7.1.3-arch1-2] (local build) Copyright (C) 2002-25, Bruce Allen, Christian Franke, [www.smartmontools.org](https://www.smartmontools.org) Read NVMe Identify Controller failed: scsi error unsupported scsi opcode

That -d sntasmedia type isn't matching correctly for this device. Try letting smartctl auto-detect, or try a couple of common alternatives for USB-NVMe bridges:

bash

sudo smartctl -a /dev/sdb

If that fails or shows generic garbage, try:

bash

sudo smartctl -a -d sat /dev/sdb

If that also fails:

bash

sudo smartctl -a -d usbjmicron /dev/sdb

And if none of those work cleanly, ask smartctl to scan and tell you what it thinks the device is:

bash

sudo smartctl --scan

sudo smartctl -a -d test /dev/sdb

The RTL9210 is known to be finicky about SAT/NVMe passthrough — some firmware revisions just don't expose SMART data properly over USB no matter what -d type you use. If all of these fail, don't spend too much more time on it — the UAS quirk fix is the higher-value action here. Let's prioritize:

- Apply the UAS-disable quirk (from my previous message), rebuild initramfs, reboot.

- Re-test with pacman upgrade or dual rsync.

- Circle back to SMART data as a secondary check, not a blocker.

Did you get a chance to add the quirk yet?

[admin@archlinux-nvme Downloads]$ sudo smartctl -a /dev/sdb smartctl 7.5 2025-04-30 r5714 [x86_64-linux-7.1.3-arch1-2] (local build) Copyright (C) 2002-25, Bruce Allen, Christian Franke, [www.smartmontools.org](https://www.smartmontools.org) === START OF INFORMATION SECTION === Model Number: Samsung SSD 980 PRO 2TB Serial Number: S69ENX0W703372E Firmware Version: 5B2QGXA7 PCI Vendor/Subsystem ID: 0x144d IEEE OUI Identifier: 0x002538 Total NVM Capacity: 2,000,398,934,016 [2.00 TB] Unallocated NVM Capacity: 0 Controller ID: 6 NVMe Version: 1.3 Number of Namespaces: 1 Namespace 1 Size/Capacity: 2,000,398,934,016 [2.00 TB] Namespace 1 Utilization: 1,561,633,468,416 [1.56 TB] Namespace 1 Formatted LBA Size: 512 Namespace 1 IEEE EUI-64: 002538 b731b48d81 Local Time is: Tue Jul 28 13:11:09 2026 IST Firmware Updates (0x16): 3 Slots, no Reset required Optional Admin Commands (0x0017): Security Format Frmw_DL Self_Test Optional NVM Commands (0x0057): Comp Wr_Unc DS_Mngmt Sav/Sel_Feat Timestmp Log Page Attributes (0x0f): S/H_per_NS Cmd_Eff_Lg Ext_Get_Lg Telmtry_Lg Maximum Data Transfer Size: 128 Pages Warning Comp. Temp. Threshold: 82 Celsius Critical Comp. Temp. Threshold: 85 Celsius Supported Power States St Op Max Active Idle RL RT WL WT Ent_Lat Ex_Lat 0 + 8.49W - - 0 0 0 0 0 0 1 + 4.48W - - 1 1 1 1 0 200 2 + 3.18W - - 2 2 2 2 0 1000 3 - 0.0400W - - 3 3 3 3 2000 1200 4 - 0.0050W - - 4 4 4 4 500 9500 Supported LBA Sizes (NSID 0x1) Id Fmt Data Metadt Rel_Perf 0 + 512 0 0 === START OF SMART DATA SECTION === SMART overall-health self-assessment test result: PASSED SMART/Health Information (NVMe Log 0x02, NSID 0xffffffff) Critical Warning: 0x00 Temperature: 39 Celsius Available Spare: 100% Available Spare Threshold: 10% Percentage Used: 0% Data Units Read: 11,579,343 [5.92 TB] Data Units Written: 15,808,226 [8.09 TB] Host Read Commands: 391,018,790 Host Write Commands: 519,931,643 Controller Busy Time: 389 Power Cycles: 3,184 Power On Hours: 689 Unsafe Shutdowns: 2,193 Media and Data Integrity Errors: 0 Error Information Log Entries: 0 Warning Comp. Temperature Time: 0 Critical Comp. Temperature Time: 0 Temperature Sensor 1: 39 Celsius Temperature Sensor 2: 39 Celsius Warning: NVMe Get Log truncated to 0x200 bytes, 0x200 bytes zero filled Error Information (NVMe Log 0x01, 16 of 64 entries) No Errors Logged Warning: NVMe Get Log truncated to 0x200 bytes, 0x034 bytes zero filled Self-test Log (NVMe Log 0x06, NSID 0xffffffff) Self-test status: No self-test in progress No Self-tests Logged

This is genuinely good news — the drive itself is completely healthy:

- Critical Warning: 0x00 — no critical issues flagged

- Available Spare: 100%, threshold 10% — plenty of headroom

- Percentage Used: 0% — basically no wear despite 8TB written

- Media and Data Integrity Errors: 0 — zero actual data corruption at the NAND/controller level

- Temperature: 39°C — nowhere near the 82°C warning threshold, so thermal throttling isn't currently happening either

- Error Information Log: No Errors Logged

The one number that stands out and confirms our whole theory: Unsafe Shutdowns: 2,193

That's a huge count relative to Power Cycles: 3,184 — meaning a very large fraction of the times this drive has powered up, it was preceded by an unsafe shutdown (i.e., power/connection was lost without a clean unmount). That's the NVMe controller's own internal counter corroborating exactly what we suspected: the USB link is dropping repeatedly, not gracefully, and the drive has been enduring this over and over. Each of those unsafe shutdowns is a potential moment for filesystem-level corruption (which is what you're seeing as BTRFS errors), even though the NAND itself remains totally healthy.

So to summarize where we are:

- Drive/NAND: healthy. Not a dying SSD, not media wear, not the actual chip failing.

- Root cause: the RTL9210 USB bridge is disconnecting under sustained heavy I/O, self-hosted or not, and has been doing so repeatedly (2,193 times) over this drive's life.

- Every one of those disconnects is a small risk of BTRFS metadata corruption, which is what's accumulating and eventually surfacing as bigger problems.

Next step — apply the UAS quirk fix now, this is the priority action:

bash

echo "options usb-storage quirks=0bda:9210:u" | sudo tee /etc/modprobe.d/rtl9210-quirk.conf

sudo mkinitcpio -P

sudo reboot

After reboot, verify UAS is actually disabled:

bash

dmesg | grep -i uas

lsusb -t

You should see usb-storage as the driver, not uas, for the Realtek device.

Then stress-test deliberately: run the same pacman upgrade or dual-rsync workload that crashed it before, and watch dmesg -w live in another terminal. If it survives without the -71 errors, the quirk fixed it.

Also worth doing once stable: run a filesystem check to see if any of those 2,193 unsafe shutdowns left latent corruption:

bash

sudo btrfs check /dev/sdb9

sudo btrfs check /dev/sdb8

(read-only check, no --repair yet — just to see if it reports anything, since we don't want to blind-repair on a live root fs without more care)

Want to go ahead and apply the quirk + reboot now, or check the btrfs state first?

[admin@archlinux-nvme ~]$ sudo dmesg | grep -i uas lsusb -t [sudo] password for admin: [ 2.666425] usb 2-1: UAS is ignored for this device, using usb-storage instead [ 2.676311] scsi host1: uas [ 2.676523] usbcore: registered new interface driver uas /: Bus 001.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/2p, 480M |__ Port 002: Dev 002, If 0, Class=Wireless, Driver=btusb, 12M |__ Port 002: Dev 002, If 1, Class=Wireless, Driver=btusb, 12M /: Bus 002.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/1p, 10000M |__ Port 001: Dev 002, If 0, Class=Mass Storage, Driver=usb-storage, 5000M /: Bus 003.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/3p, 480M |__ Port 001: Dev 002, If 0, Class=Imaging, Driver=[none], 480M /: Bus 004.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/2p, 10000M |__ Port 002: Dev 002, If 0, Class=Mass Storage, Driver=uas, 5000M /: Bus 005.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/1p, 480M |__ Port 001: Dev 002, If 0, Class=Video, Driver=uvcvideo, 480M |__ Port 001: Dev 002, If 1, Class=Video, Driver=uvcvideo, 480M |__ Port 001: Dev 002, If 2, Class=Application Specific Interface, Driver=[none], 480M [admin@archlinux-nvme ~]$

Since system is mounted:

[admin@archlinux-nvme ~]$ lsblk NAME MAJ:MIN RM SIZE RO TYPE MOUNTPOINTS sda 8:0 1 114.6G 0 disk ├─sda1 8:1 1 512M 0 part └─sda2 8:2 1 114.1G 0 part sdb 8:16 0 1.8T 0 disk ├─sdb1 8:17 0 599M 0 part ├─sdb2 8:18 0 6G 0 part ├─sdb3 8:19 0 50G 0 part ├─sdb4 8:20 0 40G 0 part ├─sdb5 8:21 0 1.6T 0 part /mnt/nvme ├─sdb6 8:22 0 512M 0 part /boot ├─sdb7 8:23 0 6G 0 part [SWAP] ├─sdb8 8:24 0 55G 0 part / └─sdb9 8:25 0 38.5G 0 part /home zram0 253:0 0 3.5G 0 disk [SWAP] nvme0n1 259:0 0 476.9G 0 disk ├─nvme0n1p1 259:1 0 260M 0 part ├─nvme0n1p2 259:2 0 16M 0 part ├─nvme0n1p3 259:3 0 175.5G 0 part /mnt/hdd2 ├─nvme0n1p4 259:4 0 300.5G 0 part /mnt/hdd └─nvme0n1p5 259:5 0 623M 0 part

[admin@archlinux-nvme ~]$ sudo btrfs check /dev/sdb9 sudo btrfs check /dev/sdb8 Opening filesystem to check... ERROR: /dev/sdb9 is currently mounted, use --force if you really intend to check the filesystem Opening filesystem to check... ERROR: /dev/sdb8 is currently mounted, use --force if you really intend to check the filesystem

Good news on two fronts:

1. UAS is already disabled for your RTL9210 — look at this line:

usb 2-1: UAS is ignored for this device, using usb-storage instead

And in lsusb -t, Bus 002 (your Samsung drive) shows Driver=usb-storage, not uas. This means the kernel's built-in quirk database already blacklists UAS for this specific RTL9210 revision — you don't even need to add the manual quirk file, it's already active by default. Good to know: this crash is happening despite UAS already being disabled, which shifts my earlier theory.

(The other device you see with Driver=uas on Bus 004 is your SanDisk drive, unrelated to this issue.)

2. This means the crash isn't a raw UAS bug — it's happening at the more basic usb-storage/BOT level, or it's a power/enumeration issue independent of UAS vs BOT. That points back toward:

- USB-C port power delivery limits under sustained load, or

- The RTL9210 enclosure's own power/thermal handling under heavy sustained multi-queue I/O, even in BOT mode

- Or a marginal cable/connector under load

On btrfs check — you're right, you can't check a live-mounted root/home filesystem safely. Options:

A) Read-only check is still blocked while mounted — btrfs check fundamentally needs the filesystem unmounted (or at least not actively written to). You'd need to boot from another medium (your other USB drive) and run it against sdb8/sdb9 while this system isn't running from them.

B) Simpler first step — just check for existing errors without full check, using dmesg/journal instead, since you're currently mounted and stable:

bash

sudo journalctl -k | grep -i btrfs | grep -i error

This shows if there are ongoing/recent errors on the currently running filesystem without needing to unmount anything.

C) Full offline check — boot from your other external USB drive (like before), then:

bash

sudo btrfs check /dev/sdb9

sudo btrfs check /dev/sdb8

This is the thorough option and matches your earlier successful "boot from other drive" test scenario.

Given UAS is already off, I'd actually now prioritize a live capture of a crash in progress over further static checks. Next time you trigger the crash (pacman upgrade or dual rsync), run this in a separate terminal/workspace first:

bash

sudo dmesg -w | tee ~/crash_capture.txt

That'll capture the lead-up to the disconnect (link speed drops, reset attempts, power negotiation messages) not just the aftermath — which will tell us definitively whether it's power delivery, thermal, or something else in the RTL9210's handling of sustained load.

Want to do that live capture next time, or check the journalctl btrfs error history first since you're already mounted and stable right now?

[admin@archlinux-nvme ~]$ sudo journalctl -k | grep -i btrfs | grep -i error [admin@archlinux-nvme ~]$

No errors currently — clean right now, which fits: the corruption only shows up when the USB link actually drops mid-write, not as some ongoing background issue.

So here's where things stand:

- Drive (NAND/controller): fully healthy — SMART data confirms this.

- UAS: already disabled by kernel quirk — so that's not the lever left to pull.

- No lingering btrfs errors in the current boot — the corruption is transient, tied directly to the disconnect events, not ongoing damage.

- Root cause still unconfirmed precisely — could be USB-C port power delivery ceiling, RTL9210 thermal/power handling under sustained load even in BOT mode, or a marginal cable.

Next real step: catch it live. Since static checks are clean, we need to see what happens in the seconds before a crash, not after. Next time you're about to run a workload that previously crashed it (pacman -Syyu, or dual rsync):

- Open a separate terminal/workspace first and run:

bash

sudo dmesg -w | tee ~/crash_capture.txt

Leave it running and visible.

- Then trigger the workload as normal (pacman upgrade or dual rsync).

- If/when it crashes, that terminal will have captured the lead-up — look for things like:

  - Link speed renegotiation (new SuperSpeed USB device re-appearing)

  - Power-related messages

  - xhci_hcd port reset attempts

  - Any over-current warnings

- If it survives the freeze (screen doesn't fully lock), you can cat ~/crash_capture.txt afterward. If it hard-freezes like before, you'll need to photograph the screen again — but this time we'll have the detailed pre-crash sequence, not just the aftermath cascade.

Also, one thing worth trying in parallel, cheap and easy: if you have literally any other USB-C cable, even a phone charging cable that supports data (many don't, so check), swap it for this test. Marginal cables are a very common cause of exactly this "works fine at idle, drops under sustained load" pattern.

Do you have a spare USB-C cable to test with, or should we just go with the live dmesg capture on the next heavy workload?

[ 0.000000] Linux version 7.1.3-arch1-2 (linux@archlinux) (gcc (GCC) 16.1.1 20260625, GNU ld (GNU Binutils) 2.46.1) #1 SMP PREEMPT_DYNAMIC Thu, 09 Jul 2026 19:55:55 +0000 [ 0.000000] Command line: initrd=\initramfs-linux.img root=UUID=3dccdd27-818e-4d57-90ae-4dfacb28d932 rw [ 0.000000] BIOS

pasted

Long output:

This is just a clean, healthy boot log — no errors, no disconnects, nothing crash-related in here. Good sign that the drive currently boots cleanly, but it doesn't tell us anything new about the freeze itself since it doesn't cover the crash window.

One small thing worth noting from this log though:

[ 12.015055] FAT-fs (sdb6): Volume was not properly unmounted. Some data may be corrupt. Please run fsck.

That's your EFI/boot FAT partition (sdb6) — it also shows signs of an unsafe shutdown, consistent with the pattern we already found via Unsafe Shutdowns: 2,193 in the SMART data. Not urgent, but worth an fsck.fat -a /dev/sdb6 next time you're booted from elsewhere.

Also confirmed from this log: your NVMe (RTL9210, bus 2) and the SanDisk drive (bus 4) are on separate USB host controllers (0000:03:00.3 vs 0000:03:00.4), so they're not competing for the same controller bandwidth — that rules out one thing.

What we actually need now is a log that spans the crash itself, not a fresh boot. This log ends at 26 seconds — nowhere near a pacman upgrade or rsync session. To actually catch the failure:

- Boot up normally, get to a stable desktop.

- Open a terminal and run:

bash

sudo dmesg -w | tee ~/crash_capture.txt

Leave this running and visible in its own workspace.
3. In another workspace, start your normal heavy workload (pacman -Syyu, or your two parallel rsyncs).
4. Watch the dmesg terminal. If you see anything about USB resets, link speed drops, or the error -71 messages starting to appear, that's the live sequence we need.
5. If the system hard-freezes before you can copy the file out, you'll need to photograph the screen again like before — but if it doesn't fully lock (some freezes leave the mouse/cursor responsive briefly), try to quickly journalctl -f or copy the file to another location before it dies completely.

Alternatively, if you want a safety net that survives even a hard freeze: set up persistent journal logging to disk (if not already) so journalctl -b after a reboot has full detail:

bash

sudo mkdir -p /var/log/journal

sudo systemd-tmpfiles --create --prefix /var/log/journal

sudo systemctl restart systemd-journald

Then after a crash and reboot, run:

bash

sudo journalctl -b -1 -k | tail -200

to see the tail end of the previous boot's kernel log — which should capture the moments right before the freeze, without needing to catch it live in a terminal.

Want to set up persistent journaling now, so the next crash is captured automatically without you needing to babysit a live terminal?

But meanshile to cleanup , I tried bleachbit as root, but failing:

[root@archlinux-nvme ~]# bleachbit Error: system.dns_cache: Function: DNS cache Traceback (most recent call last): File "/usr/share/bleachbit/Worker.py", line 92, in execute for ret in cmd.execute(self.really_delete): ~~~~~~~~~~~^^^^^^^^^^^^^^^^^^^^ File "/usr/share/bleachbit/Command.py", line 185, in execute func_ret = self.func() File "/usr/share/bleachbit/Unix.py", line 968, in flush_dns raise RuntimeError( f'Command: {args}\nReturn code: {rc}\nStdout: {stdout}\nStderr: {stderr}') RuntimeError: Command: ['resolvectl', 'flush-caches'] Return code: 1 Stdout: Stderr: Failed to flush caches: Could not activate remote peer 'org.freedesktop.resolve1': activation request failed: unknown unit [root@archlinux-nvme ~]# bleachbit Error: system.dns_cache: Function: DNS cache Traceback (most recent call last): File "/usr/share/bleachbit/Worker.py", line 92, in execute for ret in cmd.execute(self.really_delete): ~~~~~~~~~~~^^^^^^^^^^^^^^^^^^^^ File "/usr/share/bleachbit/Command.py", line 185, in execute func_ret = self.func() File "/usr/share/bleachbit/Unix.py", line 968, in flush_dns raise RuntimeError( f'Command: {args}\nReturn code: {rc}\nStdout: {stdout}\nStderr: {stderr}') RuntimeError: Command: ['resolvectl', 'flush-caches'] Return code: 1 Stdout: Stderr: Failed to flush caches: Could not activate remote peer 'org.freedesktop.resolve1': activation request failed: unknown unit

This is a minor, unrelated issue — BleachBit's "DNS cache" cleaner is trying to flush systemd-resolved's cache via resolvectl, but your system isn't running systemd-resolved (probably using NetworkManager's internal DNS or plain resolv.conf instead), so the unit doesn't exist and the call fails.

Quick fix: just deselect that cleaner

In BleachBit's GUI, uncheck System → DNS Cache before running the clean. That option is only meaningful if you're running systemd-resolved.

If running BleachBit from CLI instead:

bash

bleachbit --list

shows all cleaners; you can exclude the specific one:

bash

bleachbit -c --preset

or explicitly list the cleaners you want (excluding system.dns_cache), e.g.:

bash

bleachbit -c system.cache system.tmp system.trash

To confirm what DNS setup you're actually running (optional, just curiosity/troubleshooting):

bash

systemctl status systemd-resolved

resolvectl status

If that says the unit isn't found or inactive, that confirms it — you're not using resolved, so this cleaner option simply doesn't apply to your setup and is safe to just skip.

This has no relation to your NVMe/USB freezing issue — it's purely a BleachBit config mismatch. Once you uncheck that one item, the rest of the cleanup should run fine. Let me know if any other cleaner throws similar errors.

[admin@archlinux-nvme Downloads]$ systemctl status systemd-resolved resolvectl status ○ systemd-resolved.service - Network Name Resolution Loaded: loaded (/usr/lib/systemd/system/systemd-resolved.service; disabled; preset: enabled) Active: inactive (dead) TriggeredBy: ○ systemd-resolved-varlink.socket ○ systemd-resolved-monitor.socket Docs: man:systemd-resolved.service(8) man:org.freedesktop.resolve1(5) https://systemd.io/WRITING_NETWORK_CONFIGURATION_MANAGERS https://systemd.io/WRITING_RESOLVER_CLIENTS Failed to connect to service /run/systemd/resolve/io.systemd.Resolve: No such file or directory [admin@archlinux-nvme Downloads]$

That confirms it exactly: systemd-resolved.service is loaded but inactive/disabled, so resolvectl has nothing to talk to — hence the failure in BleachBit's DNS cache cleaner.

Two options, take whichever fits how you actually manage DNS:

Option A (recommended, simplest): just skip that cleaner in BleachBit
Since you're not using systemd-resolved, that cleaner doesn't apply to you at all. In the GUI, uncheck System → DNS Cache, and BleachBit will run cleanly without touching it. Nothing to fix system-side.

Option B: if you actually want systemd-resolved active (only do this if you specifically want it, e.g. for DNS-over-TLS, split DNS, etc.)

bash

sudo systemctl enable --now systemd-resolved

sudo ln -sf /run/systemd/resolve/stub-resolv.conf /etc/resolv.conf

But if your networking currently works fine (NetworkManager handling DNS directly, which is the common Arch default), there's no need to do this — it'd just be extra complexity for no benefit.

I'd go with Option A — just deselect that one cleaner and move on with your cleanup. Everything else in BleachBit should run normally.

But that did solve bleachbit failure / crash:

[root@archlinux-nvme ~]# bleachbit [root@archlinux-nvme ~]# bleachbit [root@archlinux-nvme ~]#

Good — no more traceback, exits cleanly now that DNS Cache is unchecked.

Quick sanity check on that output though: BleachBit opening and closing instantly with no GUI staying open, no errors, and returning straight to prompt — is that expected for you (e.g. you ran a --clean preset or automated command), or were you expecting the GUI window to stay open? If you were expecting the window to launch and stay visible, worth confirming that's actually happening, since running it as root can sometimes cause GUI display issues (DISPLAY/XAUTHORITY not passed through to root's environment) even though the process itself exits without error.

If the GUI is opening fine and you're just running a preview/clean and it's returning to prompt as expected, you're all set.

Once you're done with cleanup, the priority items are still:

- Set up persistent journaling so the next NVMe freeze gets fully logged

- Run the crash-capture test (dmesg -w in one workspace, heavy workload in another)

- Try that spare cable if you have one

Want to move forward with persistent journaling setup now?

Gui stays open, but when I click clean button on gui it crashes.

Good to know — that's likely still the same underlying issue if DNS Cache got re-checked (BleachBit sometimes resets selections between runs), or it could be a different cleaner failing this time. Let's see the actual error rather than guess.

Run it from a terminal so you can see the crash output live:

bash

bleachbit

(leave the terminal open, don't background it)

Then click Clean in the GUI, let it crash, and paste whatever traceback appears in that terminal — it'll look similar to the DNS cache one, with a Traceback (most recent call last): block naming the specific function that failed.

Also worth checking directly: is DNS Cache still unchecked in the GUI right now? If BleachBit reset your preferences or you're using a different preset/profile than last time, that could easily be the same bug resurfacing. Quick way to check without re-running the crash:

bash

grep -i dns_cache ~/.config/bleachbit/bleachbit.ini

or as root, since you're running it as root:

bash

grep -i dns_cache /root/.config/bleachbit/bleachbit.ini

Paste the terminal traceback once you have it and we'll fix whatever's actually failing this time.

### Images hidden in shared chats

Some issues still:

That message at top — "This option is protected. To bypass protection, e..." — is BleachBit's built-in safety warning, not a crash. It's truncated in the screenshot, but the full message is normally something like:

"This option is protected. To bypass protection, hold Ctrl and Shift while clicking Clean."

So this isn't actually an error — it's BleachBit refusing to run a protected cleaner (likely System → Cache or similar) unless you explicitly override it.

Also, I can see DNS cache is still checked in this screenshot — so if it crashes when you click Clean normally, that's likely still the same resolvectl issue. Uncheck it again:

- Under System, uncheck DNS cache

To proceed:

- First uncheck DNS cache (still ticked in your screenshot).

- Then either:

  - Click Clean normally, and if that protected-option warning appears again, hold Ctrl+Shift while clicking Clean to bypass it, or

  - Manually uncheck whichever protected item is triggering that warning first, and click Clean normally.

If it still crashes after unchecking DNS cache, run it from terminal as before:

bash

bleachbit

and click Clean, then paste the full traceback that appears in that terminal — that'll tell us exactly which function failed this time, since the GUI itself won't show a full Python traceback, only a generic crash/close.

With the options clicked above let us go command line and then see what messages appear

Good approach — running via CLI will show the full traceback directly in terminal instead of a silent GUI crash.

Based on your screenshot, here's the equivalent CLI clean with those same options selected (everything checked except DNS cache, Empty space, Deep scan items, and VIM History):

bash

sudo bleachbit -c bash.history bash.temp_files \

journald.clean \

pacman.cache \

snap.disabled \

system.cache system.clipboard system.custom system.desktop_entry \

system.memory system.recent_documents system.rotated_logs \

system.tmp system.trash

Run that and paste whatever output/traceback appears — this will show us exactly which cleaner (if any) fails, function name and all, same as the DNS cache error earlier.

A couple of notes on the command:

- I excluded system.dns_cache since we know that one fails without systemd-resolved active.

- If any option name doesn't match exactly, BleachBit will just complain "not found" for that one specific item — not a big deal, we can fix names as needed.

- If you want to first just confirm current available cleaner IDs and their exact names (in case Arch's bleachbit build differs slightly):

bash

bleachbit --list

paste that list if the above command errors with unknown cleaner IDs.

Go ahead and run it — let's see what comes back.

[root@archlinux-nvme ~]# sudo bleachbit -c bash.history bash.temp_files \ journald.clean \ pacman.cache \ snap.disabled \ system.cache system.clipboard system.custom system.desktop_entry \ system.memory system.recent_documents system.rotated_logs \ system.tmp system.trash journalctl --vacuum-time=1 0B Clipboard 0B Recent documents list 0B Delete 8.2kB /root/.bash_history Exception while running operation 'bash': 'Unknown option 'temp_files'' Traceback (most recent call last): File "/usr/share/bleachbit/Worker.py", line 382, in run_operations for _dummy in self.clean_operation(operation): ~~~~~~~~~~~~~~~~~~~~^^^^^^^^^^^ File "/usr/share/bleachbit/Worker.py", line 185, in clean_operation for cmd in backends[operation].get_commands(option_id): ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~^^^^^^^^^^^ File "/usr/share/bleachbit/Cleaner.py", line 94, in get_commands raise RuntimeError(f"Unknown option '{option_id}'") RuntimeError: Unknown option 'temp_files' Wiping the swap device /dev/sdb7. Wiping the swap device /dev/zram0. Error: system.memory: Function: Memory Traceback (most recent call last): File "/usr/share/bleachbit/Worker.py", line 92, in execute for ret in cmd.execute(self.really_delete): ~~~~~~~~~~~^^^^^^^^^^^^^^^^^^^^ File "/usr/share/bleachbit/Command.py", line 188, in execute for func_ret in self.func(): ~~~~~~~~~^^ File "/usr/share/bleachbit/Memory.py", line 311, in wipe_memory wipe_swap_linux(devices, proc_swaps) ~~~~~~~~~~~~~~~^^^^^^^^^^^^^^^^^^^^^ File "/usr/share/bleachbit/Memory.py", line 287, in wipe_swap_linux wipe_contents(device, truncate=False) ~~~~~~~~~~~~~^^^^^^^^^^^^^^^^^^^^^^^^ File "/usr/share/bleachbit/Wipe.py", line 256, in wipe_contents f = wipe_write() File "/usr/share/bleachbit/Wipe.py", line 206, in wipe_write f = open(path, 'wb') OSError: [Errno 16] Device or resource busy: '/dev/zram0' Disk space recovered: 8.2kB Files deleted: 1 Special operations: 3 Errors: 2

Good — this actually caught two separate, minor issues. Neither is a serious problem:

1. bash.temp_files — my mistake, wrong option ID
That's on me — I guessed a option name that doesn't exist in your BleachBit version. Not a real bug, just remove it. bash.history alone is the valid one.

2. system.memory — real error, easy to explain

OSError: [Errno 16] Device or resource busy: '/dev/zram0'

BleachBit's "Memory" cleaner tries to wipe swap devices by opening them directly for writing — but /dev/zram0 is currently active as swap (in use by the running system), so the kernel refuses to let anything else open it for direct writes while it's live. This is expected behavior, not a bug in your system — you can't safely wipe active swap while it's being used, since the OS is actively paging into it.

This is exactly the kind of "protected" operation the GUI warned about earlier — it's inherently risky to run while the system is live, which is likely why it's marked protected by default.

Simplest fix: just uncheck "Memory" under System in the GUI. It's not something you typically want to run on a live system anyway — it can cause slowdowns or instability if the kernel is actively using that swap when BleachBit tries to zero it out.

Good news: despite those 2 errors, everything else in your list succeeded:

Disk space recovered: 8.2kB

Files deleted: 1

Special operations: 3

Errors: 2

Journal vacuum ran, clipboard/recent docs cleared, bash history deleted — it's working correctly overall.

Recommended cleanup command going forward (dropping the two problem items):

bash

sudo bleachbit -c bash.history \

journald.clean \

pacman.cache \

snap.disabled \

system.cache system.clipboard system.custom system.desktop_entry \

system.recent_documents system.rotated_logs \

system.tmp system.trash

That should run clean with zero errors. Want to run that, or just go back to the GUI now that you know which two boxes to leave unchecked (Memory, DNS cache)?

But I am still worried that even after cleaning my root drive still shows 73% used:

Filesystem Size Used Avail Use% Mounted on /dev/sdb8 55G 40G 16G 73% / devtmpfs 3.5G 0 3.5G 0% /dev tmpfs 3.5G 62M 3.5G 2% /dev/shm efivarfs 256K 59K 193K 24% /sys/firmware/efi/efivars tmpfs 1.4G 1.5M 1.4G 1% /run /dev/nvme0n1p3 176G 92G 84G 53% /mnt/hdd2 /dev/nvme0n1p4 301G 171G 130G 57% /mnt/hdd /dev/sdb6 511M 81M 431M 16% /boot tmpfs 3.5G 8.0M 3.5G 1% /tmp /dev/sdb9 39G 15G 24G 38% /home /dev/sdb5 1.7T 331G 1.4T 20% /mnt/nvme tmpfs 717M 36K 717M 1% /run/user/1000 none 1.0M 0 1.0M 0% /run/credentials/systemd-journald.service none 1.0M 0 1.0M 0% /run/credentials/systemd-resolved.service

40G used on a 55G root is genuinely a lot for a base Arch install — BleachBit's cache/log cleanup only nets you kilobytes/megabytes typically; the real disk hogs are usually elsewhere. Let's find out what's actually eating the space.

1. Quick top-level breakdown:

bash

sudo du -h --max-depth=1 / 2>/dev/null | sort -rh | head -20

This shows which top-level directories are the biggest consumers (usually /var, /usr, or /home if it's mounted separately — which yours is, so it won't count here).

2. Since this is BTRFS, check for snapshots eating space silently — this is very commonly the actual culprit and BleachBit won't touch it at all:

bash

sudo btrfs filesystem usage /

sudo btrfs subvolume list /

If you have Snapper or Timeshift running, old snapshots can silently consume tens of GB even though du on visible files looks fine — because BTRFS keeps old data blocks alive as long as a snapshot references them.

3. Check pacman cache size specifically (very common bloat source, holds every installed package version):

bash

du -sh /var/cache/pacman/pkg/

If large, clean old versions (keep latest only):

bash

sudo pacman -Sc

or remove all cached packages entirely:

bash

sudo pacman -Scc

4. Check for orphaned packages taking space unnecessarily:

bash

pacman -Qtdq

If it lists packages, you can remove them:

bash

sudo pacman -Rns $(pacman -Qtdq)

5. Check journal size (even after vacuum, sometimes binary journal files linger):

bash

sudo journalctl --disk-usage

6. Check /var/log for huge log files directly:

bash

sudo du -h /var/log/ --max-depth=1 | sort -rh

Run step 1 and step 2 first — those two will most likely reveal exactly where the 40G is hiding, especially if BTRFS snapshots are involved since that's invisible to normal cleanup tools like BleachBit entirely.

[root@archlinux-nvme ~]# sudo du -h --max-depth=1 / 2>/dev/null | sort -rh | head -20 ^C [root@archlinux-nvme ~]# sudo btrfs filesystem usage / sudo btrfs subvolume list / Overall: Device size: 55.00GiB Device allocated: 44.02GiB Device unallocated: 10.98GiB Device missing:

pasted

Cleaned up a lot!

Great result — from 40G down to 24G used, freeing up 16G. The pacman package cache was indeed the main culprit, exactly as suspected.

Two things worth flagging from that output though:

1. Those "Error reading fd 7" messages on download-XXXXXX files

These are leftover partial/incomplete download files from interrupted pacman transactions — likely from that earlier session where you had the stale db.lck issue (pacman got killed mid-download at some point, possibly during one of the NVMe disconnects). They're harmless orphaned temp files, and pacman couldn't clean them via its normal cache-clean logic because they're not valid package archives, just partial downloads.

You can safely remove them directly:

bash

sudo find /var/cache/pacman/pkg/ -name "download-*" -delete

Then re-run:

bash

sudo pacman -Sc

to confirm the cache is now clean with no errors.

2. Given your NVMe's history of disconnecting under load, these leftover partial downloads are actually further circumstantial evidence of past drive dropouts corrupting in-progress writes — consistent with everything we found in the SMART data (Unsafe Shutdowns: 2,193) and the BTRFS EIO cascade in your crash screenshot. Nothing to worry about now, just another data point confirming the same root cause.

Current state:

- Root: 43% used (down from 73%) — healthy headroom now (31G free)

- Package cache cleaned

- No BTRFS snapshots hogging space (confirmed via btrfs subvolume list — just portables/machines, nothing heavy)

Once you clear those stray download-* files, you're in good shape on the disk-space front. That leaves the actual NVMe freeze investigation as the remaining open item — happy to pick that back up (persistent journaling + live crash capture) whenever you're ready.

[root@archlinux-nvme ~]# sudo find /var/cache/pacman/pkg/ -name "download-*" -delete find: cannot delete ‘/var/cache/pacman/pkg/download-6Gu3gM’: Directory not empty find: cannot delete ‘/var/cache/pacman/pkg/download-snLQYm’: Directory not empty find: cannot delete ‘/var/cache/pacman/pkg/download-dLjzr2’: Directory not empty find: cannot delete ‘/var/cache/pacman/pkg/download-0gViv1’: Directory not empty

Ah — these are actually directories, not files, so plain -delete won't remove their contents recursively. Use rm -rf instead:

bash

sudo find /var/cache/pacman/pkg/ -maxdepth 1 -name "download-*" -exec rm -rf {} +

Or simpler, since you already know they're safe to remove:

bash

sudo rm -rf /var/cache/pacman/pkg/download-*

Then confirm they're gone:

bash

ls /var/cache/pacman/pkg/ | grep download

(should return nothing)

And re-check disk usage:

bash

df -h /

These being directories rather than files is a bit unusual for pacman's cache — normally cached packages are just .pkg.tar.zst files. Directories named download-XXXXXX are typically leftover temp/staging folders from parallel downloads (pacman/reflector sometimes stage downloads in temp dirs before finalizing them), again consistent with interrupted transactions — likely from the same crash events.

[root@archlinux-nvme ~]# sudo find /var/cache/pacman/pkg/ -maxdepth 1 -name "download-*" -exec rm -rf {} + [root@archlinux-nvme ~]# ls /var/cache/pacman/pkg/ | grep download [root@archlinux-nvme ~]# df -h / Filesystem Size Used Avail Use% Mounted on /dev/sdb8 55G 23G 32G 42% / [root@archlinux-nvme ~]#

All cleaned up — 42% used, 32G free. Good spot to be in.

Recap of where things stand overall:

Disk space: ✅ resolved — root went from 73% → 42% used.

The actual NVMe/USB freeze issue: still open. To recap what we know:

- Drive health (SMART): perfectly healthy, 0% wear, no media errors

- Bridge: RTL9210B-CG, UAS already disabled by kernel quirk automatically

- Crash trigger: heavy sustained write load (pacman upgrade OR parallel rsync) while self-hosted on this same drive

- Same load without self-hosting (booted from elsewhere) = no crash

- Root cause still points to: USB-C port power/bandwidth ceiling or RTL9210 enclosure struggling under combined baseline OS I/O + heavy write burst

- Confirmed corroborating evidence: Unsafe Shutdowns: 2,193 in SMART data, plus those orphaned partial pacman downloads — both point to a history of the link dropping mid-transfer

Next step whenever you're ready: set up persistent journald logging so the next freeze gets fully captured automatically, without needing a live terminal watching:

bash

sudo mkdir -p /var/log/journal

sudo systemd-tmpfiles --create --prefix /var/log/journal

sudo systemctl restart systemd-journald

Then after any future crash + reboot:

bash

sudo journalctl -b -1 -k | tail -200

This shows the tail end of the previous boot's kernel log — capturing the lead-up to the freeze even if you never got a live terminal open at the time.

Want to set that up now, or take a break on this and revisit when you next hit the freeze?