From: https://claude.ai/chat/6f0ba0ee-42d2-4980-9b19-f6855d14f736

# you asked: I just ins

message time: 2026-10-11 16:19:17

I just installed new git kernel on fedora:

admin@fedora-rh-usb:~$ uname -r 7.3.0-0.rc6.261009gaf32da41b032.55.fc46.x86_64

Problem is this. while booting the kernel, it just freezed at logo screen. Culprit is this nvme drive:

sdb           8:16   0   1.8T  0 disk ├─sdb1        8:17   0   599M  0 part ├─sdb2        8:18   0     6G  0 part ├─sdb3        8:19   0    50G  0 part ├─sdb4        8:20   0    40G  0 part ├─sdb5        8:21   0   1.6T  0 part ├─sdb6        8:22   0   512M  0 part ├─sdb7        8:23   0     6G  0 part ├─sdb8        8:24   0    55G  0 part └─sdb9        8:25   0  38.5G  0 part

See this is my fstab but it has no mount from that external drive. When it was freezing I just removed physically that external nvme drive from usb port and the booting process finished and I could come here. What is the issu? After booting, I again plugged in that nvme drive to that port and it is showing up also.

admin@fedora-rh-usb:~$ cat /etc/fstab

# /etc/fstab

# Created by anaconda on Wed Feb  4 13:44:32 2026

# Accessible filesystems, by reference, are maintained under '/dev/disk/'.

# See man pages fstab(5), findfs(8), mount(8) and/or blkid(8) for more info.

# After editing this file, run 'systemctl daemon-reload' to update systemd

# units generated from this file.

UUID=8a093eed-ccc2-4f7e-8cd0-a9e43b720f12 / btrfs defaults 0 0 UUID=D3CA-75B5 /boot/efi vfat umask=0077,shortname=winnt 0 2 UUID=01DAD08525577F90  /mnt/hdd   ntfs3  ro,defaults,nofail,x-systemd.device-timeout=10  0 0 UUID=4AAA9C11AA9BF821  /mnt/hdd2  ntfs3  ro,defaults,nofail,x-systemd.device-timeout=10  0 0

# claude response: Good diagn

Good diagnosis so far. The drive not being in /etc/fstab is actually a red herring — the freeze is happening before fstab is even consulted, during the initramfs/early-boot phase. A few likely causes:

## Why it freezes even without an fstab entry

### 1. Btrfs multi-device scan (most likely)

Your root filesystem is btrfs. During initramfs, btrfs scans all block devices looking for btrfs member volumes. With the new kernel, this scan may be hanging or timing out on your USB NVMe — possibly a regression in the USB storage or btrfs layer.

### 2. udev settling

systemd-udev-settle in the initramfs waits for all devices to stabilize. If the USB NVMe takes too long (or triggers a driver quirk in the new kernel), it stalls the entire boot.

### 3. New kernel USB/NVMe regression

Kernel 7.3.0-rc6 is a release candidate — driver regressions in USB mass storage or NVMe-over-USB are not uncommon at this stage.

## Diagnostics

Check the previous failed boot's journal:

bash
 journalctl -b -1 | grep -i -E "timeout|error|usb|nvme|btrfs|udev|hung|freeze"

See what was happening around the freeze time:

bash
 journalctl -b -1 --no-pager | tail -100

Check dmesg from the bad boot:

bash
 journalctl -b -1 -k | grep -i -E "usb|nvme|error|warn|timeout" | head -60

## Workarounds to try

Option A — Add a kernel boot parameter (edit GRUB temporarily at boot):

At the GRUB menu, press e, find the linux line, and append:

rd.udev.settle_timeout=5

This caps how long initramfs waits for udev. If btrfs scan is the culprit, try also:

rd.btrfs.udev.settle_timeout=5

Option B — Disable btrfs from scanning all devices (permanent):

bash
 sudo grubby --args="rd.btrfs.scan=0" --update-kernel=ALL
 # or more targeted:
 sudo grubby --args="rd.udev.settle_timeout=10" --update-kernel=ALL

Option C — Add a udev rule to ignore the drive during boot:

Find the drive's ID:

bash
 udevadm info --query=all --name=/dev/sdb | grep -E "ID_SERIAL|ID_MODEL|ID_VENDOR"

Then create /etc/udev/rules.d/99-ignore-usb-nvme.rules to tag it as SYSTEMD_WANTS="" during early boot.

Option D — Simplest: just keep the drive unplugged during boot

Since the drive works fine after boot, this is a reasonable day-to-day workaround while the kernel issue gets sorted, especially since it's an RC kernel.

## Recommended next step

Run journalctl -b -1 | grep -i "btrfs\|udev\|usb\|settle\|timeout" and share what it shows — that'll pinpoint whether it's btrfs scanning, udev settling, or a USB driver hang, and we can target the fix precisely.

# you asked: But I want

message time: 2026-10-11 16:22:33

But I want that drive to be scanned it has btrfs and ntfs both - it has two linux systems - fedora rawhide and arch linux.

# claude response: That's an

That's an important distinction. Since your host system's root is also btrfs, the initramfs must scan to confirm which btrfs volumes belong to which filesystem — you can't skip it. The problem is the scan is hanging on the USB NVMe specifically in this RC kernel.

## What's actually happening

The external NVMe has btrfs partitions → initramfs btrfs udev scan tries to read them early → something in the USB ↔ NVMe bridge layer hangs in kernel 7.3.0-rc6 before the full driver stack is ready.

The drive works after boot because by then the full USB stack is properly initialized.

## First, gather info from the bad boot

bash
 # Last failed boot kernel messages
 journalctl -b -1 -k --no-pager | tail -80
 
 # Specifically USB + btrfs during that boot
 journalctl -b -1 -k | grep -iE "usb|nvme|btrfs|xhci|uas|uas_"

bash
 # Get your USB NVMe bridge chip ID
 lsusb | grep -i nvme
 # or just
 lsusb

bash
 # Get the exact USB device details
 udevadm info --query=all --name=/dev/sdb | grep -E "ID_|DRIVER|VENDOR|MODEL"

## Most targeted fix — USB storage quirks

Many USB-NVMe enclosures have known bugs. Linux has a quirks system for exactly this. First find your device's VID:PID:

bash
 lsusb
 # example output: Bus 002 Device 003: ID 174c:55aa ASMedia Technology Inc. ...
 #                                        ^^^^ ^^^^
 #                                        VID  PID

Then add a boot parameter (edit in GRUB with e at boot, or make permanent):

bash
 # For UAS (USB Attached SCSI) — most modern USB-NVMe enclosures use this
 usb-storage.quirks=174c:55aa:u
 # 'u' = disable UAS, fall back to plain usb-storage
 
 # Or force old usb-storage driver entirely:
 usb-storage.quirks=174c:55aa:b

To make permanent:

bash
 sudo grubby --args="usb-storage.quirks=VID:PID:u" --update-kernel=ALL
 sudo grub2-mkconfig -o /boot/grub2/grub.cfg

## Alternative — increase initramfs timeout without skipping scan

bash
 # At GRUB boot line, append:
 rd.udev.settle_timeout=30 rootdelay=15

This gives the USB stack more time to fully initialize before btrfs scanning begins — often enough to fix USB NVMe hangs.

## The real fix

This is a kernel regression in an RC — worth reporting:

bash
 # Check if it worked on the previous kernel
 sudo dnf list --installed kernel
 # Boot into the previous kernel from GRUB → Advanced to confirm the drive works fine there

If the old kernel boots fine with the drive plugged in, this is a clear regression to report at bugzilla.redhat.com or bugzilla.kernel.org. Share the journalctl -b -1 -k output — the USB VID:PID and the exact hang point will pinpoint the driver.

Share the lsusb output and journalctl -b -1 -k | grep -iE "usb|nvme|btrfs|xhci|uas" and I can give you the exact quirk parameter for your enclosure.

# you asked: Shows like

message time: 2026-10-11 16:23:57

Shows like this but my current booted drive is usb so all messages capture wrong drive?

Oct 11 16:42:02 fedora-rh-usb kernel: audit: type=1131 audit(1791717122.636:663): pid=1 uid=0 auid=4294967295 ses=4294967295 subj=system_u:system_r:init_t:s0 msg='unit=lvm2-monitor comm="systemd" exe="/usr/lib/systemd/systemd" hostname=? addr=? terminal=? res=success' Oct 11 16:42:02 fedora-rh-usb systemd-shutdown[1]: Syncing filesystems and block devices. Oct 11 16:42:03 fedora-rh-usb systemd-shutdown[1]: Sending SIGTERM to remaining processes... Oct 11 16:42:03 fedora-rh-usb systemd-journald[839]: Received SIGTERM from PID 1 (systemd-shutdow).

# claude response: Good catch

Good catch. You have two USB devices here:

- Your boot drive (running Fedora right now) — also USB

- The external NVMe (sdb) that caused the freeze

And the freeze happened in initramfs — before systemd/journald even started — so it likely wrote nothing to the journal. That's why -b -1 just shows a normal shutdown.

## What to do instead

Since the drive is plugged in right now and working, grab its info from the current boot:

bash
 # See how the drive came up this boot
 dmesg | grep -iE "usb|nvme|uas|xhci|sdb" | head -60

bash
 # Get VID:PID of the NVMe enclosure
 lsusb

bash
 # Detailed USB info for sdb specifically
 udevadm info --query=all --name=/dev/sdb | grep -E "ID_VENDOR|ID_MODEL|ID_SERIAL|DRIVER|ID_USB"

bash
 # Check if it's using UAS or fallback usb-storage driver
 cat /sys/block/sdb/device/../../driver/module/parameters/* 2>/dev/null || \
 lsusb -t | grep -A3 "NVMe\|Mass"

## Why the current boot succeeded but initramfs didn't

| Phase | USB stack state |
| --- | --- |
| initramfs (freeze point) | Partial — xHCI just started, UAS driver not fully ready |
| After boot (now) | Full USB stack, UAS negotiated properly |

The drive takes too long to respond during UAS negotiation in early initramfs, and the new kernel's timeout is tighter or there's a regression in the xHCI/UAS path.

Share the lsusb and udevadm info output — once we have the VID:PID, we can add the exact usb-storage.quirks= parameter to make it work reliably during boot.

# you asked: [ 0.392457

message time: 2026-10-11 16:26:49

[    0.392457] ACPI: bus type USB registered [    0.392457] usbcore: registered new interface driver usbfs [    0.392457] usbcore: registered new interface driver hub [    0.392457] usbcore: registered new device driver usb [    0.781896] xhci_hcd 0000:03:00.3: xHCI Host Controller [    0.781948] xhci_hcd 0000:03:00.3: new USB bus registered, assigned bus number 1 [    0.782372] xhci_hcd 0000:03:00.3: hcc params 0x0120ffc5 hci version 0x120 quirks 0x0000000200000010 [    0.782808] xhci_hcd 0000:03:00.3: xHCI Host Controller [    0.782856] xhci_hcd 0000:03:00.3: new USB bus registered, assigned bus number 2 [    0.782860] xhci_hcd 0000:03:00.3: Host supports USB 3.1 Enhanced SuperSpeed [    0.782890] usb usb1: New USB device found, idVendor=1d6b, idProduct=0002, bcdDevice= 7.03 [    0.782940] usb usb1: New USB device strings: Mfr=3, Product=2, SerialNumber=1 [    0.782943] usb usb1: Product: xHCI Host Controller [    0.782945] usb usb1: Manufacturer: Linux 7.3.0-0.rc6.261009gaf32da41b032.55.fc46.x86_64 xhci-hcd [    0.782947] usb usb1: SerialNumber: 0000:03:00.3 [    0.783105] hub 1-0:1.0: USB hub found [    0.783682] usb usb2: We don't know the algorithms for LPM for this host, disabling LPM. [    0.783684] usb usb2: New USB device found, idVendor=1d6b, idProduct=0003, bcdDevice= 7.03 [    0.783719] usb usb2: New USB device strings: Mfr=3, Product=2, SerialNumber=1 [    0.783722] usb usb2: Product: xHCI Host Controller [    0.783724] usb usb2: Manufacturer: Linux 7.3.0-0.rc6.261009gaf32da41b032.55.fc46.x86_64 xhci-hcd [    0.783726] usb usb2: SerialNumber: 0000:03:00.3 [    0.783850] hub 2-0:1.0: USB hub found [    0.784316] xhci_hcd 0000:03:00.4: xHCI Host Controller [    0.784366] xhci_hcd 0000:03:00.4: new USB bus registered, assigned bus number 3 [    0.784755] xhci_hcd 0000:03:00.4: hcc params 0x0120ffc5 hci version 0x120 quirks 0x0000000200000010 [    0.785188] xhci_hcd 0000:03:00.4: xHCI Host Controller [    0.785243] xhci_hcd 0000:03:00.4: new USB bus registered, assigned bus number 4 [    0.785254] xhci_hcd 0000:03:00.4: Host supports USB 3.1 Enhanced SuperSpeed [    0.785291] usb usb3: New USB device found, idVendor=1d6b, idProduct=0002, bcdDevice= 7.03 [    0.785384] usb usb3: New USB device strings: Mfr=3, Product=2, SerialNumber=1 [    0.785390] usb usb3: Product: xHCI Host Controller [    0.785395] usb usb3: Manufacturer: Linux 7.3.0-0.rc6.261009gaf32da41b032.55.fc46.x86_64 xhci-hcd [    0.785399] usb usb3: SerialNumber: 0000:03:00.4 [    0.785668] hub 3-0:1.0: USB hub found [    0.786569] usb usb4: We don't know the algorithms for LPM for this host, disabling LPM. [    0.786572] usb usb4: New USB device found, idVendor=1d6b, idProduct=0003, bcdDevice= 7.03 [    0.786610] usb usb4: New USB device strings: Mfr=3, Product=2, SerialNumber=1 [    0.786612] usb usb4: Product: xHCI Host Controller [    0.786614] usb usb4: Manufacturer: Linux 7.3.0-0.rc6.261009gaf32da41b032.55.fc46.x86_64 xhci-hcd [    0.786616] usb usb4: SerialNumber: 0000:03:00.4 [    0.786750] hub 4-0:1.0: USB hub found [    0.787405] xhci_hcd 0000:04:00.0: xHCI Host Controller [    0.787473] xhci_hcd 0000:04:00.0: new USB bus registered, assigned bus number 5 [    0.788782] xhci_hcd 0000:04:00.0: USB0 root hub has no ports [    0.788800] xhci_hcd 0000:04:00.0: hcc params 0x0110ffc5 hci version 0x120 quirks 0x0000000200000010 [    0.789187] usb usb5: New USB device found, idVendor=1d6b, idProduct=0002, bcdDevice= 7.03 [    0.789230] usb usb5: New USB device strings: Mfr=3, Product=2, SerialNumber=1 [    0.789232] usb usb5: Product: xHCI Host Controller [    0.789235] usb usb5: Manufacturer: Linux 7.3.0-0.rc6.261009gaf32da41b032.55.fc46.x86_64 xhci-hcd [    0.789237] usb usb5: SerialNumber: 0000:04:00.0 [    0.789408] hub 5-0:1.0: USB hub found [    0.789942] usbcore: registered new interface driver usbserial_generic [    0.789950] usbserial: USB Serial support registered for generic [    0.798681] usbcore: registered new interface driver usbhid [    0.798685] usbhid: USB HID core driver [    1.023481] usb 3-2: new high-speed USB device number 2 using xhci_hcd [    1.024158] usb 1-2: new full-speed USB device number 2 using xhci_hcd [    1.031473] usb 5-1: new high-speed USB device number 2 using xhci_hcd [    1.149267] usb 3-2: New USB device found, idVendor=0bda, idProduct=9210, bcdDevice=20.01

Bus 001 Device 001: ID 1d6b:0002 Linux Foundation 2.0 root hub Bus 001 Device 002: ID 0bda:b85d Realtek Semiconductor Corp. Bluetooth Radio Bus 002 Device 001: ID 1d6b:0003 Linux Foundation 3.0 root hub Bus 002 Device 002: ID 0781:55a9 SanDisk Corp.  SanDisk 3.2Gen1 Bus 003 Device 001: ID 1d6b:0002 Linux Foundation 2.0 root hub Bus 003 Device 003: ID 0bda:9210 Realtek Semiconductor Corp. RTL9210 M.2 NVME Adapter Bus 004 Device 001: ID 1d6b:0003 Linux Foundation 3.0 root hub Bus 005 Device 001: ID 1d6b:0002 Linux Foundation 2.0 root hub Bus 005 Device 002: ID 0408:5487 Quanta Computer, Inc. HP True Vision FHD Camera

E: ID_MODEL=Samsung_SSD_980_PRO_2TB E: ID_MODEL_ENC=Samsung\x20SSD\x20980\x20PRO\x202TB\x20\x20\x20\x20\x20\x20\x20\x20\x20\x20\x20\x20\x20\x20\x20\x20\x20 E: ID_SERIAL=Samsung_SSD_980_PRO_2TB_S69ENX0W703372E E: ID_SERIAL_SHORT=S69ENX0W703372E E: ID_USB_MODEL=RTL9210B-CG E: ID_USB_MODEL_ENC=RTL9210B-CG\x20\x20\x20\x20\x20 E: ID_USB_MODEL_ID=9210 E: ID_USB_SERIAL=Realtek_RTL9210B-CG_012345679039-0:0 E: ID_USB_SERIAL_SHORT=012345679039 E: ID_USB_VENDOR=Realtek E: ID_USB_VENDOR_ENC=Realtek\x20 E: ID_USB_VENDOR_ID=0bda E: ID_USB_REVISION=1.00 E: ID_USB_TYPE=disk E: ID_USB_INSTANCE=0:0 E: ID_USB_INTERFACES=:080650: E: ID_USB_INTERFACE_NUM=00 E: ID_USB_DRIVER=usb-storage

lsusb -t | grep -A3 "NVMe|Mass" |__ Port 001: Dev 002, If 0, Class=Mass Storage, Driver=uas, 5000M /:  Bus 003.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/3p, 480M |__ Port 002: Dev 003, If 0, Class=Mass Storage, Driver=usb-storage, 480M /:  Bus 004.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/2p, 10000M /:  Bus 005.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/1p, 480M |__ Port 001: Dev 002, If 0, Class=Video, Driver=uvcvideo, 480M

# claude response: Now the pi

Now the picture is very clear. Two key findings:

## What the data reveals

Your boot drive (SanDisk):

- 0781:55a9 → Bus 002 → Driver=uas → 5000M ✅

Your external NVMe (Samsung 980 PRO in Realtek RTL9210B-CG enclosure):

- 0bda:9210 → Bus 003 → Driver=usb-storage → 480M ⚠️

### Problem 1 — Only running at USB 2.0 speed (480M)

The RTL9210B-CG should be doing 5000M or 10000M. It's negotiating at USB 2.0 speed. With 9 partitions (including btrfs), scanning this drive at 480M during initramfs is extremely slow and triggers a timeout/hang.

First, verify the cable and port:

bash
 # Is it plugged into a USB 3.x port? Check which port is 5000M+
 lsusb -t
 # Bus 004 has 10000M but no storage device on it
 # Try plugging the RTL9210 enclosure into the port your SanDisk uses

### Problem 2 — RTL9210B-CG has known initramfs quirks

This is a very well-known problematic chip. Even though it's already on usb-storage (not UAS), it still has timeout issues during early boot.

## Fix 1 — Increase udev settle timeout (quick test)

At GRUB menu, press e, add to the kernel line:

rd.udev.settle_timeout=30

If this fixes the boot → make it permanent:

bash
 sudo grubby --args="rd.udev.settle_timeout=30" --update-kernel=ALL

## Fix 2 — Add RTL9210 quirk (permanent)

bash
 # 'u' disables UAS + sets safe fallback flags for usb-storage
 sudo grubby --args="usb-storage.quirks=0bda:9210:u" --update-kernel=ALL
 sudo grub2-mkconfig -o /boot/grub2/grub.cfg

## Fix 3 — Tell dracut about the device so initramfs handles it properly

bash
 # Create dracut config to add udev settle time
 sudo nano /etc/dracut.conf.d/usb-nvme.conf

Add:

add_drivers+=" usb-storage "
 install_items+=" /etc/udev/rules.d/ "

Then rebuild initramfs:

bash
 sudo dracut --force --kver 7.3.0-0.rc6.261009gaf32da41b032.55.fc46.x86_64

## Also try: fix the 480M issue first

bash
 # What port is the RTL9210 currently on?
 lsusb -t
 # Bus 003 is only 480M capable — that's a USB 2.0 bus on your machine
 # Bus 002 and 004 are SuperSpeed — try those ports

If the enclosure jumps to 5000M, the btrfs scan will complete fast enough and the boot hang may disappear entirely without any other changes.

# you asked: But the dr

message time: 2026-10-11 16:29:38

But the drive I  booted is on ctype port - it is usb. Which fix to apply and then try rebooting with drive plugged in on that usb port?

sudo grubby --args="rd.udev.settle_timeout=30" --update-kernel=ALL

# claude response: Apply **bo

Apply both together in one command — more coverage, one reboot:

bash
 sudo grubby --args="rd.udev.settle_timeout=30 usb-storage.quirks=0bda:9210:u" --update-kernel=ALL

Then verify it was applied:

bash
 sudo grubby --info=DEFAULT | grep args

You should see both parameters in the output. Then reboot with the NVMe plugged in:

bash
 sudo reboot

Why both:

- rd.udev.settle_timeout=30 — gives initramfs 30s instead of the default ~5s to wait for the slow 480M drive to enumerate all 9 partitions

- usb-storage.quirks=0bda:9210:u — tells the kernel the RTL9210B-CG chip needs safe fallback flags from the very start

If it still hangs, come back and we'll look at rebuilding the initramfs with dracut. But this combo should do it.

# you asked: There are

message time: 2026-10-11 16:30:44

There are other diagnostics also:

admin@fedora-rh-usb:~$ # Is it plugged into a USB 3.x port? Check which port is 5000M+ lsusb -t

# Bus 004 has 10000M but no storage device on it

# Try plugging the RTL9210 enclosure into the port your SanDisk uses

/:  Bus 001.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/2p, 480M |__ Port 002: Dev 002, If 0, Class=Wireless, Driver=btusb, 12M |__ Port 002: Dev 002, If 1, Class=Wireless, Driver=btusb, 12M /:  Bus 002.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/1p, 10000M |__ Port 001: Dev 002, If 0, Class=Mass Storage, Driver=uas, 5000M /:  Bus 003.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/3p, 480M |__ Port 002: Dev 003, If 0, Class=Mass Storage, Driver=usb-storage, 480M /:  Bus 004.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/2p, 10000M /:  Bus 005.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/1p, 480M |__ Port 001: Dev 002, If 0, Class=Video, Driver=uvcvideo, 480M |__ Port 001: Dev 002, If 1, Class=Video, Driver=uvcvideo, 480M |__ Port 001: Dev 002, If 2, Class=Application Specific Interface, Driver=[none], 480M admin@fedora-rh-usb:~$ # Bus 004 has 10000M but no storage device on it

# Try plugging the RTL9210 enclosure into the port your SanDisk uses

/:  Bus 001.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/2p, 480M |__ Port 002: Dev 002, If 0, Class=Wireless, Driver=btusb, 12M |__ Port 002: Dev 002, If 1, Class=Wireless, Driver=btusb, 12M /:  Bus 002.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/1p, 10000M |__ Port 001: Dev 002, If 0, Class=Mass Storage, Driver=uas, 5000M /:  Bus 003.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/3p, 480M |__ Port 002: Dev 003, If 0, Class=Mass Storage, Driver=usb-storage, 480M /:  Bus 004.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/2p, 10000M /:  Bus 005.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/1p, 480M |__ Port 001: Dev 002, If 0, Class=Video, Driver=uvcvideo, 480M |__ Port 001: Dev 002, If 1, Class=Video, Driver=uvcvideo, 480M |__ Port 001: Dev 002, If 2, Class=Application Specific Interface, Driver=[none], 480M bash: /:: No such file or directory bash: syntax error near unexpected token |' bash: syntax error near unexpected token |' bash: /:: No such file or directory bash: syntax error near unexpected token |' bash: /:: No such file or directory bash: syntax error near unexpected token |' bash: /:: No such file or directory bash: /:: No such file or directory bash: syntax error near unexpected token |' bash: syntax error near unexpected token |' bash: syntax error near unexpected token `|' admin@fedora-rh-usb:~~$ sudo grubby --args="rd.udev.settle_timeout=30 usb-storage.quirks=0bda:9210:u" --update-kernel=ALL admin@fedora-rh-usb:~~$ sudo grubby --info=DEFAULT | grep args args="ro rhgb quiet $tuned_params rd.udev.settle_timeout=30 usb-storage.quirks=0bda:9210:u"

# claude response: The args a

The args are correctly set. ✅

args="ro rhgb quiet $tuned_params rd.udev.settle_timeout=30 usb-storage.quirks=0bda:9210:u"

One more thing before rebooting — your RTL9210 NVMe is on Bus 003 which is a 480M (USB 2.0) port. Bus 004 has a 10000M root hub but nothing plugged in.

Try plugging the RTL9210 NVMe into that Bus 004 port (the other USB-C/USB-A port on your machine) — it will get 10x faster enumeration during initramfs, which reduces the hang risk further.

Then reboot:

bash
 sudo reboot

What to expect:

- If it boots → great, the fix worked. We can then investigate getting the drive to 5000M/10000M speed permanently

- If it still hangs → unplug, boot normally, come back and we'll rebuild the initramfs with dracut as the next step

The bash errors you saw were just because you accidentally pasted the lsusb -t output into the terminal — those aren't real errors, ignore them.

# you asked: I tried th

message time: 2026-10-11 16:34:06

I tried that other port and now booted successfully but of course with little longer time lag compared to normal boot time which is very fast.

Can we diagnose further? Or I have to compromise?

# claude response: Great news

Great news! Now let's diagnose the remaining lag. Run these:

bash
 # How long did boot actually take?
 systemd-analyze
 
 # Which stage took longest?
 systemd-analyze blame | head -20
 
 # Critical chain
 systemd-analyze critical-chain

bash
 # Is the NVMe now running at proper speed on the new port?
 lsusb -t | grep -A2 "9210\|Mass"

bash
 # Any udev settle delays logged?
 journalctl -b 0 -k | grep -iE "settle|udev|rtl9210|usb.*9210|btrfs.*scan"

Share those outputs and we can pinpoint exactly where the lag is coming from.

The likely culprits are:

| Cause | Fix |
| --- | --- |
| Still running at 480M | Move to 10000M port → RTL9210 negotiates USB 3.x |
| btrfs scan taking long with 9 partitions | Tune btrfs udev rules |
| rd.udev.settle_timeout=30 is too generous | Reduce to exact needed value once we know |
| RTL9210 slow power-on response | Firmware quirk, cable, or enclosure issue |

The 30s timeout we set is a ceiling — it waits up to 30s but stops as soon as udev settles. The actual delay is whatever the drive actually needs. The numbers from systemd-analyze will tell us exactly how much time is being lost and where.

# you asked: graphical.

message time: 2026-10-11 16:35:01

graphical.target reached after 13.097s in userspace. 11.027s dev-disk-by\x2duuid-8a093eed\x2dccc2\x2d4f7e\x2d8cd0\x2da9e43b720f12.device 11.027s dev-disk-by\x2dpath-pci\x2d0000:03:00.3\x2dusb\x2d0:1:1.0\x2dscsi\x2d0:0:0:0\x2dpart-by\x2dpartuuid-727e81a4\x2d02.device 11.027s dev-disk-by\x2dpath-pci\x2d0000:03:00.3\x2dusb\x2d0:1:1.0\x2dscsi\x2d0:0:0:0\x2dpart-by\x2duuid-8a093eed\x2dccc2\x2d4f7e\x2d8cd0\x2da9e43b720f12.device 11.027s dev-disk-by\x2dpartuuid-727e81a4\x2d02.device 11.027s dev-disk-by\x2dpath-pci\x2d0000:03:00.3\x2dusbv3\x2d0:1:1.0\x2dscsi\x2d0:0:0:0\x2dpart2.device 11.027s dev-disk-by\x2did-usb\x2dUSB_SanDisk_3.2Gen1_03021206111725204508\x2d0:0\x2dpart2.device 11.027s dev-disk-by\x2dpath-pci\x2d0000:03:00.3\x2dusb\x2d0:1:1.0\x2dscsi\x2d0:0:0:0\x2dpart-by\x2dpartnum-2.device 11.027s dev-disk-by\x2ddiskseq-1\x2dpart2.device 11.027s dev-sda2.device 11.027s dev-disk-by\x2dpath-pci\x2d0000:03:00.3\x2dusb\x2d0:1:1.0\x2dscsi\x2d0:0:0:0\x2dpart2.device 11.027s sys-devices-pci0000:00-0000:00:08.1-0000:03:00.3-usb2-2\x2d1-2\x2d1:1.0-host1-target1:0:0-1:0:0:0-block-sda-sda2.device 11.026s dev-disk-by\x2dpartuuid-727e81a4\x2d01.device 11.026s dev-disk-by\x2did-usb\x2dUSB_SanDisk_3.2Gen1_03021206111725204508\x2d0:0\x2dpart1.device 11.026s dev-disk-by\x2dpath-pci\x2d0000:03:00.3\x2dusb\x2d0:1:1.0\x2dscsi\x2d0:0:0:0\x2dpart-by\x2dpartnum-1.device 11.026s dev-sda1.device 11.026s dev-disk-by\x2dpath-pci\x2d0000:03:00.3\x2dusb\x2d0:1:1.0\x2dscsi\x2d0:0:0:0\x2dpart1.device 11.026s dev-disk-by\x2duuid-D3CA\x2d75B5.device 11.026s dev-disk-by\x2dpath-pci\x2d0000:03:00.3\x2dusb\x2d0:1:1.0\x2dscsi\x2d0:0:0:0\x2dpart-by\x2dlabel-BOOTEFI.device 11.026s dev-disk-by\x2dpath-pci\x2d0000:03:00.3\x2dusb\x2d0:1:1.0\x2dscsi\x2d0:0:0:0\x2dpart-by\x2dpartuuid-727e81a4\x2d01.device 11.026s dev-disk-by\x2dpath-pci\x2d0000:03:00.3\x2dusb\x2d0:1:1.0\x2dscsi\x2d0:0:0:0\x2dpart-by\x2duuid-D3CA\x2d75B5.device The time when unit became active or started is printed after the "@" character. The time the unit took to start is printed after the "+" character.

graphical.target @13.097s └─multi-user.target @13.097s └─rsyslog.service @12.997s +98ms └─network-online.target @12.983s └─NetworkManager-wait-online.service @8.579s +4.402s └─NetworkManager.service @5.336s +3.239s └─network-pre.target @5.334s └─firewalld.service @5.334s └─basic.target @5.312s └─dbus-broker.service @5.263s +48ms └─dbus.socket @5.249s +764us └─sysinit.target @5.247s └─systemd-binfmt.service @4.232s +1.014s └─proc-sys-fs-binfmt_misc.mount @4.911s +332ms └─proc-sys-fs-binfmt_misc.automount @1.913s admin@fedora-rh-usb:$ # Is the NVMe now running at proper speed on the new port? lsusb -t | grep -A2 "9210|Mass" |__ Port 001: Dev 002, If 0, Class=Mass Storage, Driver=uas, 5000M /:  Bus 003.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/3p, 480M |__ Port 001: Dev 002, If 0, Class=Mass Storage, Driver=usb-storage, 480M /:  Bus 004.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/2p, 10000M /:  Bus 005.Port 001: Dev 001, Class=root_hub, Driver=xhci_hcd/1p, 480M admin@fedora-rh-usb:$ # Any udev settle delays logged? journalctl -b 0 -k | grep -iE "settle|udev|rtl9210|usb.9210|btrfs.scan" Oct 11 22:34:37 fedora kernel: Command line: BOOT_IMAGE=(hd1,msdos2)/boot/vmlinuz-7.3.0-0.rc6.261009gaf32da41b032.55.fc46.x86_64 root=UUID=8a093eed-ccc2-4f7e-8cd0-a9e43b720f12 ro rhgb quiet rd.udev.settle_timeout=30 usb-storage.quirks=0bda:9210:u Oct 11 22:34:37 fedora kernel: Kernel command line: BOOT_IMAGE=(hd1,msdos2)/boot/vmlinuz-7.3.0-0.rc6.261009gaf32da41b032.55.fc46.x86_64 root=UUID=8a093eed-ccc2-4f7e-8cd0-a9e43b720f12 ro rhgb quiet rd.udev.settle_timeout=30 usb-storage.quirks=0bda:9210:u Oct 11 22:34:37 fedora kernel: usb 3-1: New USB device found, idVendor=0bda, idProduct=9210, bcdDevice=20.01 Oct 11 22:34:37 fedora kernel: usb 3-1: Product: RTL9210B-CG Oct 11 22:34:37 fedora systemd[1]: Listening on systemd-udevd-kernel.socket - udev Kernel Socket. Oct 11 22:34:37 fedora systemd[1]: Listening on systemd-udevd-varlink.socket - udev Varlink Socket. Oct 11 22:34:38 fedora kernel: usb-storage 3-1:1.0: Quirks match for vid 0bda pid 9210: 800000 Oct 11 22:34:39 fedora kernel: scsi 0:0:0:0: Direct-Access     Realtek  RTL9210B-CG      1.00 PQ: 0 ANSI: 6 Oct 11 22:34:44 fedora kernel: Lockdown: (udev-worker): direct PCI access is restricted; see man kernel_lockdown.7 Oct 11 22:34:45 fedora kernel: BTRFS: device fsid 8a093eed-ccc2-4f7e-8cd0-a9e43b720f12 devid 1 transid 15906 /dev/sda2 (8:2) scanned by mount (734) Oct 11 17:04:47 fedora-rh-usb systemd[1]: Listening on systemd-udevd-varlink.socket - udev Varlink Socket. Oct 11 17:04:47 fedora-rh-usb systemd[1]: Starting systemd-udev-load-credentials.service - Load udev Rules from Credentials... Oct 11 17:04:47 fedora-rh-usb systemd[1]: Starting systemd-udev-trigger.service - Coldplug All udev Devices... Oct 11 17:04:47 fedora-rh-usb systemd[1]: Finished systemd-udev-load-credentials.service - Load udev Rules from Credentials. Oct 11 17:04:47 fedora-rh-usb kernel: audit: type=1130 audit(1791718487.419:8): pid=1 uid=0 auid=4294967295 ses=4294967295 subj=system_u:system_r:init_t:s0 msg='unit=systemd-udev-load-credentials comm="systemd" exe="/usr/lib/systemd/systemd" hostname=? addr=? terminal=? res=success'

# claude response: Good data.

Good data. Several things stand out:

## What the numbers show

1. The NVMe is STILL at 480M — wrong port again

Bus 003: RTL9210 → usb-storage, 480M  ← still USB 2.0!
 Bus 004: 10000M root hub → nothing plugged in  ← this is the one to use

You moved it to a different port but it's still a USB 2.0 port. The 10000M port (Bus 004) is still empty. Try every physical USB port until lsusb -t shows it on Bus 004.

2. The quirk IS working ✅

usb-storage 3-1:1.0: Quirks match for vid 0bda pid 9210: 800000

3. The real 11s delay is your BOOT drive (SanDisk), not the NVMe

11.027s dev-sda2.device  ← this is your root partition on SanDisk

This is the inherent cost of booting from USB — the SanDisk takes 11s to enumerate before the root filesystem can mount. This is largely unavoidable on USB boot.

4. NetworkManager is adding 4.4s unnecessarily

NetworkManager-wait-online.service @8.579s +4.402s

This can be fixed:

bash
 sudo systemctl disable NetworkManager-wait-online.service

This is safe — it just stops waiting for network before declaring boot complete.

## Summary of what's left

| Issue | Fix | Expected gain |
| --- | --- | --- |
| NVMe still at 480M | Find the Bus 004 physical port | Faster btrfs scan |
| NM wait-online | systemctl disable NetworkManager-wait-online.service | ~4s saved |
| 11s USB boot drive | Inherent to USB boot | Not fixable |

After disabling NM-wait-online, you should be down to ~9s. The 11s SanDisk wait happens in parallel with other things, so total boot to graphical should drop noticeably.

# you asked: Hey these

message time: 2026-10-11 16:36:54

Hey these are the only two usb port which I physically have - the third one is ctype from where I booted and on it now (on usb). There is another but that is for video - I forgot name of that port.

# claude response: Ah, that m

Ah, that makes sense now. Your physical layout is:

| Port | Bus | Speed | Device |
| --- | --- | --- | --- |
| USB-C | Bus 002 | 10000M | SanDisk (boot drive) |
| USB-A #1 | Bus 003 | 480M | RTL9210 NVMe |
| USB-A #2 | Bus 005 | 480M | Webcam (built-in) |
| Video port (DP/TB) | Bus 004 | 10000M | Display only |

So Bus 004 is your video/Thunderbolt port — not usable for storage. The NVMe is stuck at 480M because both USB-A ports are USB 2.0 only on your machine. Nothing to do there.

## What you CAN still improve

Disable NetworkManager wait — easy 4s saving:

bash
 sudo systemctl disable NetworkManager-wait-online.service

Lower the udev timeout to match reality — the quirk log shows the drive appeared at ~2s into boot, so 30s is overkill:

bash
 sudo grubby --args="rd.udev.settle_timeout=10 usb-storage.quirks=0bda:9210:u" --update-kernel=ALL

That's honestly the ceiling. The 11s wait is your USB boot drive — that's the fundamental bottleneck and it's irreducible on USB. You're looking at roughly ~8-9s to graphical after these two changes, which is as good as it gets booting two USB storage devices simultaneously.

# you asked: What is th

message time: 2026-10-11 16:37:52

What is that type called in layman terms I fogot that.

# claude response: You mean

You mean DisplayPort or Thunderbolt?

- DisplayPort — flat rectangular port, video only, no data

- Thunderbolt (looks like USB-C but with a ⚡ lightning bolt symbol) — can do video + data + power, but on your machine it seems to be wired only to the display controller, not the USB storage stack

Which one do you have physically on the machine? If it has a ⚡ symbol next to it, it's Thunderbolt. If it's a wider flat port, it's DisplayPort.

# you asked: Actually t

message time: 2026-10-11 16:38:51

Actually that port is used to project video and audio to projector.

# claude response: That's **H

That's HDMI then. 😄

Yeah, HDMI is video+audio out only — no data, no USB, nothing you can plug storage into. So your port situation is exactly what we mapped:

- USB-C → boot drive (SanDisk)

- USB-A → RTL9210 NVMe (stuck at 480M, nothing we can do)

- HDMI → projector/display only

So the fix we applied is the best you can get on this hardware. Just run the two remaining tweaks:

bash
 # Save ~4 seconds
 sudo systemctl disable NetworkManager-wait-online.service
 
 # Lower timeout to something more realistic
 sudo grubby --args="rd.udev.settle_timeout=10 usb-storage.quirks=0bda:9210:u" --update-kernel=ALL

And that's your optimal boot configuration for this setup. 🙂

# you asked: Yeah! corr

message time: 2026-10-11 16:39:52

Yeah! correct! I forgot hdmi! Goot catch.

admin@fedora-rh-usb:~$ # Save ~4 seconds sudo systemctl disable NetworkManager-wait-online.service

# Lower timeout to something more realistic

sudo grubby --args="rd.udev.settle_timeout=10 usb-storage.quirks=0bda:9210:u" --update-kernel=ALL [sudo] password for admin: Removed '/etc/systemd/system/network-online.target.wants/NetworkManager-wait-online.service'. admin@fedora-rh-usb:~$

# claude response: Both done.

Both done. ✅

Verify the grubby change stuck:

bash
 sudo grubby --info=DEFAULT | grep args

Should show rd.udev.settle_timeout=10 (not 30 anymore). Then reboot with the NVMe plugged in:

bash
 sudo reboot

You should see a noticeably snappier boot this time. Come back and share systemd-analyze output after and we can compare the before/after numbers! 🙂

# you asked: args="ro r

message time: 2026-10-11 16:40:18

args="ro rhgb quiet $tuned_params rd.udev.settle_timeout=10 usb-storage.quirks=0bda:9210:u"

# claude response: Perfect —

Perfect — settle_timeout=10 is in there. ✅

Now reboot with the NVMe plugged in:

bash
 sudo reboot

See you on the other side! 🚀

# you asked: Yes. Now b

message time: 2026-10-11 16:42:39

Yes. Now booting almost normal speed.

# claude response: Excellent!

Excellent! 🎉 Great result for an RC kernel boot with two USB storage devices!

## What we fixed today

| Problem | Fix Applied |
| --- | --- |
| Boot freeze with NVMe plugged in | usb-storage.quirks=0bda:9210:u |
| Excessive udev wait | rd.udev.settle_timeout=10 |
| NetworkManager adding 4s delay | systemctl disable NM-wait-online |

## One thing to keep in mind

When Fedora 46 goes stable and you upgrade to a release kernel, you can test booting without the grubby args to see if the RTL9210 regression got fixed upstream. If it boots fine, you can clean them up with:

bash
 sudo grubby --remove-args="rd.udev.settle_timeout=10 usb-storage.quirks=0bda:9210:u" --update-kernel=ALL

But for now, enjoy the fast boot! 😄

# you asked: Does this

message time: 2026-10-11 16:43:32

Does this in any way create trouble somewhere?

sudo grubby --remove-args="rd.udev.settle_timeout=10 usb-storage.quirks=0bda:9210:u" --update-kernel=ALL

# claude response: No trouble

No trouble at all. That command just removes those two parameters from the kernel boot line — it's a clean reversal of exactly what we added. It doesn't touch anything else.

And --update-kernel=ALL means it applies to all installed kernels, which is correct.

The only risk would be if you run it now while still on this RC kernel with the NVMe plugged in — then you'd be back to the freeze problem.

So just remember:

- Now → keep the args, they're helping

- After upgrading to stable Fedora 46 kernel → test without them, remove if boot is fine

That's it. No side effects, no permanent changes to anything else. ✅

# you asked: What if th

message time: 2026-10-11 16:43:48

What if that is not cone?

# claude response: If you nev

If you never remove them, nothing bad happens — they're harmless permanent parameters:

- rd.udev.settle_timeout=10 — just means initramfs waits up to 10s for devices. On a normal boot without the NVMe it settles in 1-2s anyway and moves on. No penalty.

- usb-storage.quirks=0bda:9210:u — only activates if a device with VID 0bda and PID 9210 (your specific RTL9210 chip) is detected. If the drive isn't plugged in, this parameter does absolutely nothing.

So even if you forget to remove them forever, they just sit there quietly doing nothing harmful. ✅

# you asked: also, I ju

message time: 2026-10-11 16:45:29

also, I just booted with that first port then also it booted normally. Wanted to test taht there is no absolute issues with that port as well.

# claude response: That makes

That makes sense! Now that the two fixes are in place:

- The quirk handles the RTL9210 chip properly regardless of which USB-A port

- The 10s timeout gives enough breathing room even on the slower enumeration

So both USB-A ports work fine now. The port was never the real fix — it was just a workaround we tried before applying the proper kernel parameters.

Your setup is now solid:

- ✅ Either USB-A port works with NVMe plugged in

- ✅ Boot speed is back to normal

- ✅ Drive shows up correctly after boot